DEVELOPMENT OF AN AUTOMATIC SPEECH-TO-SPEECH SYNTHESIS ARCHITECTURE FOR ENGLISH-TO-YORÙBÁ LANGUAGE USING NEURAL MACHINE TRANSLATION APPROACH

BY

SOBOLA GABRIEL OLUWATOBI

23PCK02485

Being a Ph.D departmental Proposal presented as part of the Requirements for the Award of the Degree of Doctor of Philosophy (Ph.D) in Information & Communication Engineering of the Department of Electrical and Information Engineering, Covenant University, Ota, Ogun State, Nigeria.

Supervisor(s):

Prof. Emmanuel Adetiba

Dr. Olabode Idowu-Bismark

JUNE 2025

# TABLE OF CONTENTS

TABLE OF CONTENTS	ii

LIST OF FIGURES	vi

LIST OF TABLES	vii

ABBREVIATIONS AND SYMBOLS	viii

ABSTRACT	xi

CHAPTER ONE	1

INTRODUCTION	1

1.1 Background to the study	1

1.2 Statement of the Problem	3

1.3 Aim and Objectives	4

1.3.1 Aim	4

1.3.2 Objectives	4

1.4 Scope of the Study	5

1.5 Justification for the Study	5

1.6 Expected Contributions to Knowledge	6

CHAPTER TWO	7

LITERATURE REVIEW	7

2.1 Introduction	7

2.2 Speech Production and Translation in Humans	7

2.2.1 Speech Production and Recognition	7

2.2.2 Speech Translation in Human	9

2.2.3 Low and High-Resourced Language	9

2.2.3.1 Low Resourced Language	9

2.3.3.2 High-Resourced Language	10

2.3 Overview of Automatic Speech-to-Speech Translation	10

2.3.1 Traditional Speech-to-Speech Translation	10

2.3.1.1 Probabilistic Approach to modelling ASR	10

2.3.1.2 Statistical Approach to Machine Translation	11

2.3.1.3 Concatenative Speech Synthesis	12

2.3.1.4 Statistical Parametric Speech Synthesis	12

2.3.2 Conventional Speech-to-Speech Translation	12

2.3.2.1 Automatic Speech Recognition	13

2.3.2.2 Machine Translation	14

2.3.2.3 Text-To-Speech	15

2.3.3 Neural Speech-to-Speech Translation	15

2.3.3.1 Neural Network Approach to Automatic Speech Recognition	15

2.3.3.2 Neural Network Approach to Machine Translation	16

2.3.3.3 Neural Network Approach to Modelling TTS	17

2.3.4 Direct Speech-to-Speech Translation	19

2.3.4.1 Generic Architecture	19

2.3.4.2 Speech Mel Spectrogram	20

2.3.4.3 The Fast Fourier Transform	21

2.3.4.4 Synthesizer	23

2.3.4.5 Encoder	23

2.3.4.6 Attention Mechanism	24

2.3.4.7 Decoder	24

2.3.4.8 Vocoder	24

2.4 State-of-the-Art Approaches for Speech-to-Speech Translation	25

2.4.1 Sequence-to-Sequence Learning	25

2.4.2 Encoder-Decoder Network	25

2.4.3 Attention Mechanism	27

2.4.4 Sequence-to-Sequence Modelling	27

2.4.5 Transformer	30

2.4.5.1 Self-Attention	32

2.4.5.2 Multi-Head Attention	32

2.4.5.3 Positional Encoding	33

2.4.6 Cross lingual Voice Cloning	33

2.5 Stages Involved in Neural End-To-End Approaches to Speech Translation	34

2.5.1 Data Collection	34

2.5.2 Data Analysis	34

2.5.3 Data Preprocessing	34

2.5.4 Data Cleaning	35

2.5.5 Feature Extraction Stage	35

2.5.6 Discretisation	35

2.5.7 Sequence-to-Sequence Modelling	35

2.6 Performance Metric	35

2.6.1 Bilingual Evaluation Understudy	35

2.6.2 Mean Opinion Score	37

2.6.3 Statistical Evaluation	38

2.7 Regularisation Techniques	39

2.8 Review of Related Works	39

2.8.1 Review of Literature on Dataset	39

2.8.2 Review of Literature on Speech-To-Speech Translation	41

2.8.3 Review of Literature on Utilisation of Machine Translation in The Medical World	51

2.8.4 Summary of Literature Review on Speech-To-Speech Synthesis	54

2.9 Gaps in existing knowledge	59

CHAPTER THREE	61

METHODOLOGY	61

3.1 Introduction	61

3.2 Summary of Methodology	61

3.3 The Proposed Direct Speech-to-Speech Architecture	62

3.3.1 General Overview of the Proposed Architecture	63

3.3.2 Source and Target Speeches	63

3.3.3 Mel-frequency Cepstral Coefficients	64

3.3.4 Discretisation	67

3.3.5 Encoder	67

3.3.6 Attention Mechanism	68

3.3.7 Speech Synthesizer	68

3.3.8 Decoder	68

3.4 Neural Machine Translation Method	69

3.4.1 Data Curation	69

3.4.1.1 Tools	69

3.4.1.2 Steps to Curation of Data	71

3.4.1.3 Other Steps for Data Curation	77

3.4.2 Analysis of Curated Data	78

3.4.3 Pre-Processing of the Analysed Data	78

3.4.3.1 HuBERT Discretisation	78

3.4.4 Improved Transformer Model Development	78

3.4.5 Training	79

3.4.5.1 Model Training Parameters	79

3.4.5.2 Base Model Training and Transfer Learning of Non-Medical Speech Corpus	80

3.4.5.3 Base Model Training and Transfer Learning of Medical Speech Corpus	81

3.4.6 Testing	81

3.4.7 Evaluation	82

3.4.7.1 Performance Comparison of Base and Transfer Learning Models for Non-Medical Speech	82

3.4.7.2 Performance Comparison of Base and Transfer Learning Models for Medical Speech Parallel Pair Corpus	82

3.4.8 Flow Chart of Neural Machine Translation Approach	82

3.4.8.1 Flow Chart Explained	83

3.5 Prototype Development	84

3.5.1 Software Packages	84

3.5.2 Prototype Softbot	84

3.5.3 Steps to Prototype SoftBot Development	84

3.5.3.1 SoftBot’s Functionality	84

3.5.3.2 Framework and Technology for SoftBoot	85

3.5.3.3 User Interface Design	85

3.5.3.4 Development of the frontend	86

3.5.3.5 Development of the backend	86

3.5.3.6 Writing of server-side code in Python to handle API requests and integration with speech-to-speech APIs	87

3.5.3.7 Speech-to-Speech Translation Model	88

3.5.3.8 Conversation Management	88

3.5.3.9 Prototype Testing and Evaluation	89

3.5.3.10 Maintenance and Update of the Website	89

3.6 Budget and Timeline	89

3.6.1 Research Budget	89

3.6.2 Timelines	91

3. 7 Conclusion	92

References	93

# LIST OF FIGURES

Figure 2.1: Conceptual Model of Speech Production and Speech Recognition Processes (Rabiner et al., 2007)	8

Figure 2.2: Block Diagram of an Overall Speech Recognition System (Rabiner et al., 2007b).	9

Figure 2.3: Pipeline (Probabilistic Approach) ASR (Medeiros, 2023a)	10

Figure 2.4: Different Speech Features Obtained at Different Processing Stages (Medeiros, 2023).	11

Figure 2.5: Conventional Approach to Modelling Speech-to-Speech Translation	13

Figure 2.6: A Generic Two-Stage Neural TTS Architecture (de Martos, 2022)	18

Figure 2.7: Generic Direct Speech-to-Speech Architecture (Translatotron 2) (Jia et al., 2022)	20

Figure 2.8: Spectrogram for English Audio - "I will be back soon” (Limbu, 2020)	21

Figure 2.9: Spectrogram for Swedish Audio - "Jag Kommer Tillbaka Snart" (Limbu, 2020)	21

Figure 2.10: Spectrogram for Speech Signal “Should we chase” with Hamming Window Length M = 101 for the Upper Plot and M = 401 for the Lower Plot (Rabiner & Schafer, 2007)	23

Figure 2.11: A Basic Encoder-Decoder Network using LSTM (Limbu, 2020)	26

Figure 2.12: A Basic Encoder-Decoder Network Showing the Encoded Vectors (de Martos, 2022)	28

Figure 2.13: Attention Mechanism Overview (Zhang et al., 2021)	29

Figure 2.14: The Original Transformer Architecture (Vaswani et al., 2017)	31

Figure 3.1: The Proposed Speech-To-Speech Architecture (An extension to Translatotron 2)	62

Figure 3.2: The Speech Synthesizer of Figure 3.1	62

Figure 3.3: MFCC Feature Extraction Process	64

Figure 3.4: Frame and Window Size Illustration	64

Figure 3.5: Flow Chart for the proposed Model of Speech-to-Speech Translation using Neural Network Approach	76

# LIST OF TABLES

Table 2.1: A summary Table for the Computation of BLEU score	36

Table 2.2: Summary of Literature Review on Speech-To-Speech Synthesis	54

Table 2.3: Summary of Literature Review on Speech-To-Speech Synthesis (Cont.)	55

Table 2.4: Summary of Literature Review on Speech-To-Speech Synthesis (Cont.)	56

Table 2.5: Summary of Literature Review on Speech-To-Speech Synthesis (Cont.)	57

Table 2.6: Summary of Literature Review on Speech-To-Speech Synthesis (Cont.)	58

Table 2.7: Summary of Gaps in Literature	59

Table 2.8: Summary of Gaps in Literatures (Cont.)	60

Table 3.1: Summary of proposed methodology	61

Table 3.2: Model Hyperparameters for Training	72

Table 3.3: Model Hyperparameters for Training	73

Table 3.4: Research Budget	77

Table 3.5: Research Timeline	78

# ABBREVIATIONS AND SYMBOLS

AI - Artificial Intelligence

API – Application Programming Interface

ANN - Artificial Neural Network

ASR - Automatic Speech Recognition

BERT – Bidirectional Encoder Representation of a Transformer

BLEU – Bilingual Evaluation Understudy

BLSTM - Bidirectional Long Short Time Memory

CNN - Convolutional Neural Network

CSS – Cascading Style Sheet

DL – Deep Learning

DNN - Deep Neural Network

DTFT – Discrete Time Fourier Transform

DFT – Discrete Fourier Transform

DSP – Digital Speech Processing

4IR - Fourth Industrial Revolution

FEDGEN – Federated Genomics

FFN – Feed Forward Network

FFT - Fast Fourier Transform

FMLLR - Feature-space Maximum Likelihood Linear Regression

GA - Genetic Algorithm

GAN – Generative Adversarial Network

GAU – Gated Activation Unit

GLU – Gated Linear Unit

GMM - Gaussian Mixture Model

GPT – Generative Pretrain Transformer

GPU - Graphic Processing Unit

HMM - Hidden Markov Model

HTML – Hypertext Mark-up Language

HuBERT – Hidden Unit Bidirectional Encoder Representation of a Transformer

IDLE – Integrated Development Learning Environment

IEEE - Institution of Electrical and Electronics Engineers

IPA - International Phonetic Alphabet

LASSO – Least Absolute Shrinkage and Selection Operator

LPC - Linear Predictive Coding

LPCC - Linear Predictive Cepstral Coefficients

LSTM - Long Short Time Memory

MATLAB – Matrix Laboratory

MFCCs - Mel Frequency Cepstral Coefficients

ML – Machine Learning

MLP - Multilayer Perceptron

MOS – Mean Opinion Score

MSK - Mask

MT – Machine Translation

NIST - National Institute of Standards and Technology

NLP – Natural Language Processing

NLU – Natural Language Understanding

NMT – Neural Machine Translation

PLP - Perceptual Linear Predictive

RASTA - Relative Spectral Filtering

RASTA - PLP - Relative Spectral Perceptual Linear Predictive

ReLU – Rectified Linear Unit

ROUGE-L: Recall Oriented Understudy for Gisting Evaluation

RNN - Recurrent Neural Network

RR - Recognition Rate

SDGs – Sustainable Development Goals

SOTA – State-of-the-Art

S2ST – Speech-to-Speech Translation

SR – Speech Recognition

SS – Speech Synthesis

STFT - Short Time Fourier Transform

STT – Speech-to-Text

TIMIT - Texas Instruments Massachusetts Institute of Technology

TTS – Text-to-Speech

U2UT – Unit-to-Unit Translation

UDR – Unaligned Duration Ratio

VQ - Vector Quantization

WER - Word Error Rate

WSJ – Wall Street Journal

# ABSTRACT

The speech-to-speech translation model is an area of research in speech processing applications that has gained recognition. It is mostly aided with the application of deep learning that ensures training of models on fast accelerating Graphic Processing Unit (GPU).  The model has solved language barriers, ensured real time communication between citizens of different background. Two well-known approaches for developing the model are the cascaded approach that involves Speech Recognition (SR), Natural Language Understanding (NLU), Machine Translation (MT), & Speech Synthesis (SS); and Direct Sequence- to-Sequence approach that utilises deep learning. The existing models suffer misalignment problem between input and output spectrograms, optimised speech processing algorithms, complexity of the model, unavailability or insufficient of training dataset, and cross lingual voice transfer. This proposal aims to develop a speech-to-speech translation model for English-to-Yorùba using the direct sequence-to-sequence neural machine translation approach taking discretisation as a key pre-processing algorithm that solves the complexity of the model and enhances reduction in errors generated. In this work, accented parallel corporal of English-Yorùba for general and medical domains, taking Malaria data as a benchmark would be curated by crowdsourcing individuals to record their voices. The curated dataset will be made public for further research-This is to ensure cross lingual voice transfer. Improved Translatotron 2 transformer architecture utilising discrete representation of speech as input would be developed using Torch and Torchaudio modules of Pytorch library. The model will be trained using this corporal that will be pre-processed, and same corporal would be fed to a transfer learning based model and performances of different experimental set-up and ablation studies would be carried out. To evaluate the proposed model, the BLEU score, MOS Naturalness, and MOS Similarity, as well as statistical metrics like the recall, precision, and f1 score, would be carried out, and lastly, a prototype SoftBot for translational use case would be developed and evaluated by a team of experts to be sourced.  This work addresses the United Nations Sustainable Development (SDG) Goals 3, ensuring healthy lives and well-being, and 10 – reducing inequality within and among the communities, as well as addressing the fourth industrial revolution (4IR).

Keywords: 	Attention Mechanism, Discretisation, Neural Machine Translation, Sequence-To-Sequence Modelling, Speech Synthesis, Transformer

# CHAPTER ONE

# INTRODUCTION

# 1.1 Background to the study

The translation of speech from one language to another is an area in speech processing applications that has gained recognition in recent years (Chang, X., Yan, B., Choi, K., Jung, J., Lu, Y., Maiti, S., Sharma, R., Shi, J., Tian, J., Watanabe, S., Fujita, Y., Maekaku, T., Guo, P., Cheng, Y.-F., Denisov, P., Saijo, K., & Wang, H.-H., 2023). There is also the translation of text from one language to another, known as machine translation (Hujon, A. V., Singh, T. D., & Amitab, K., 2022; Ranathunga, S., Lee, E.-S. A., Skenduli, M. P., Shekhar, R., Alam, M., & Kaur, R., 2021). The conventional approach to translating speech from one language to the target language involves the conversion of the source speech to text, natural language understanding, source language text translation to the target language text, and finally, the conversion of the target language text to the target language speech. The first step is carried out using speech recognition, the second step is an area of speech processing that is called Natural Language Processing (NLP), the third stage is Machine Translation (MT), and finally, Speech Synthesis (SS) to obtain the output target speech (Jia, Y., Weiss, R. J., Biadsy, F., Macherey, W., Johnson, M., Chen, Z., & Wu, Y., 2019; Rubenstein, P. K., Asawaroengchai, C., Nguyen, D. D., Bapna, A., Borsos, Z., Quitry, F. de C., Chen, P., Badawy, D. El, Han, W., Kharitonov, E., Muckenhirn, H., Padfield, D., Qin, J., Rozenberg, D., Sainath, T., Schalkwyk, J., Sharifi, M., Ramanovich, M. T., Tagliasacchi, M., … Frank, C., 2023). This approach involves text representations, generating enormous errors during the conversion stages (Jia, Weiss, et al., 2019; McArthur, B., & Richardson, S. D., 2023). The system also suffers from increased latency and mismatch problems (Jia, Y., Johnson, M., Macherey, W., Weiss, R. J., Cao, Y., Chiu, C.-C., Ari, N., Laurenzo, S., & Wu, Y., 2019), as well as limitations to the languages they were trained with and the inability to function well for languages with accent types, tonal, and special sounds (Barrault, L., Chung, Y.-A., Meglioli, M. C., Dale, D., Dong, N., Duquenne, P.-A., Elsahar, H., Gong, H., Heffernan, K., Hoffman, J., & others., 2023; Chen, P.-J., Tran, K., Yang, Y., Du, J., Kao, J., Chung, Y.-A., Tomasello, P., Duquenne, P.-A., Schwenk, H., Gong, H., & others., 2022; Kaur, P., Bush, L. A. M., & Shi, W., 2024).

Due to the availability of data and high computing power, an end-to-end machine-learning approach for speech-to-speech synthesis has gained popularity for quite a number of years (Limbu, S. H., 2020). This one-step approach, which involves the use of encoder/decoder block configuration with one input and one output, seems to be faster and more convenient for the generation of speech of the target language, and it is called direct Speech-to-Speech Translation (S2ST) (Rubenstein, et. al, 2023). It was first achieved by Jia et al., (2019), where Translatotron was developed. English-to-Yoruba speech-to-speech translation model using direct neural machine translation based on transformer architecture was carried out in Julius et al., (2024) where spectrograms of both the English and Yoruba were utilised yielding translation accuracy of 98 % with a noteworthy BLEU score of 70. This high BLEU score further shows the benefits of sufficient training data (200, 023 audios) utilised unlike the English-to-Hausa speech-to-speech translation model (trained on 8 hours of audios) and English-to-Yoruba speech-to-speech translation model (trained on 432 hours of audios) developed for Whisper, where the best BLEU scores of 0.5 and 1.4 were obtained for Whisper large, and Whisper large-v2 respectively (Radford, A., Kim, J. W., Xu, T., Brockman, G., McLeavey, C., & Sutskever, I., 2023). (Olagunju, O. B., Abiola, O. B., Saka-Balogun, O. Y., Badeji-Ajisafe, B., Oguntimilehin, A., Adewole, L. O., Adigun, K. A., Obamiyi, S. E., & Oluwatoki, T. G., 2024) developed English-to-Yoruba text-to-text neural machine translation model using transformer architecture taking texts as the input and target. Using this speech translation or representation approach, the paralinguistic features such as the voice, emotion, and prosody of the speaker are preserved as it is converted from one language to the target language (Jia, et al., 2019; Kharitonov, E., Copet, J., Lakhotia, K., Nguyen, T. A., Tomasello, P., Lee, A., Elkahky, A., Hsu, W.-N., Mohamed, A., Dupoux, E., & others., 2022; Limbu, 2020; Gupta et al., 2022), unlike the conventional approach where these features are lost during each of the stages involved. The direct S2ST can handle names and proper nouns that need not be translated (Jia, Weiss et al., 2019). The problem associated with it is generating parallel speech corpus data for both languages, which seems to be cumbersome compared to its text-to-text translation (Limbu, 2020). Consequently, researchers have developed various ways to generate training data to train models. These include (i) Text-to-Speech Application Programming Interface (TTS APIs) to generate speech corpus of the easily accessible text pairs of both languages of concern, (ii) Use of already developed S2ST models to obtain speech utterance of the target language from the source language (Limbu, 2020), etc. Another solution to this is to train direct speech-to-speech translation model in an unsupervised fashion. This involves the utilisation of unpaired or unparalleled datasets of both the source and target languages. This was first utilised for improved Translatotron 2 model called Translatotron 3 developed by Google researchers for English-to-Spanish and vice versa (Nachmani, E., Levkovitch, A., Ding, Y., Asawaroengchai, C., Zen, H., & Ramanovich, M. T., 2023). This unsupervised approach has also been utilised in machine translation for texts (Chauhan, S., Daniel, P., Saxena, S., & Sharma, A., 2022).

The English language, which is a Germanic language first spoken in the Early Middle Ages in England, is an official language spoken in the United Kingdom, the United States, Ireland, Australia, Canada, and New Zealand as the first language (Dogra, T., & Malik Student, R., 2024). It is also an official and second language spoken by the Commonwealth of Nations, the European Union, and some other organisations. Findings showed that the English language is the third most widely spoken native language in the world behind Mandarin Chinese and Spanish, as well as the most spoken language in the world spoken by nearly 1.5 billion speakers, which is slightly higher than 1.1 billion Mandarin Chinese speakers (Silaban, W. Y., Lestari, M., Desi, W. ;, Barumbu, N., Kawet, K., Yosua, ;, Sunkudon, A., Darmawan, ;, & Winoto, E., 2023). The Yorùbá language is the target language of focus in this work. It is a language that shares membership with the Niger-Congo language family Knowles, K. L., Patten, K. J., Durojaye, C., Dada, B. O., & McBeath, M. K., (2025); Oyewole, J. A., Oyewole, C. F., Ayomide, P. A. O., & Oyewole, J. A., (2025) and is one of Nigeria's three official languages. The Yorùbá language is also a native language of Nigeria, Benin, and Togo. It is spoken by a population of about 50 million across Southwestern Nigeria, coupled with the USA, UK, Brazil, Benin, and Togo (Bakenne & Salawu, 2025).

English language, a tool for oral and written communication, is also the language mostly utilized for accessing information, fostering international relations, and developing technology and science, just to mention a few. Most foreign personnel who are English speakers find it difficult to interact when they arrive at a nation whose natives are non-English speakers and sometimes experience language barriers, which hinders their communications. Besides, in Nigeria, for instance, English speakers face language barriers when they meet native Yoruba speakers who can not communicate in English. In the public health sector, health practitioners face difficulties communicating with patients who are Yoruba speakers or when discharging health-related information for public consumption.

This work will bridge the gap by developing the English-Yoruba language translator to help public health personnel discharge their duties. It will also address the Fourth Industrial Revolution (4IR), encompassing deep learning, AI, Internet technology, and cloud computing, by leveraging big data and cloud computing platforms to develop state-of-the-art S2ST results. At present, the United Nations aims to build a suitable and conducive world; hence, they desire to achieve 17 goals on or before 2030. Goal 3 – Ensuring healthy lives and well-being- and Goal 10 – reducing inequality within and among the community. The public and the medical facility will find the prototype softbot for educational purposes, thereby creating awareness of information in eradicating the spread of diseases. The native Yoruba speakers will not be isolated in acquiring information about health-related matters related to malaria. Also, everyone will be inclusive in acquiring malaria health-related information and utilizing the prototype Softbot for personalized learning. Via this work, the parallel English-Yoruba speech corpora will be made available; communications will be enhanced to ensure seamless and unhindered interactions between humans and machines.

# 1.2 Statement of the Problem

The exchange of information between speakers of different languages has led to difficulties in communications or inabilities to communicate due to the language diversities. In international events, various tools have been developed to translate from the speaker’s language to the respective participant languages. Speech technologies such as speech synthesis, speech recognition, speech-to-speech translation, etc., with progressively improved performance have been utilised. Nonetheless, the development of speech-to-speech synthesis has not been proportionally distributed across all world languages. Only a few languages like English, Spanish, and French (which are termed high-resource languages) have seen a revolution in speech-to-speech synthesis (Azterketa Eta Prozesamendua, H., 2023; Kim, S.-B., Lee, S.-H., & Lee, S.-W., 2024; Wadhawan, A., 2023). Most language translator kits from English to Yoruba and vice versa are mostly text-based translators due to the limited size of the available Yoruba speech corpus (Meyer, J., Adelani, D. I., Casanova, E., Öktem, A., Weber, D. W. J., Kabongo, S., Salesky, E., Orife, I., Leong, C., Ogayo, P., Emezue, C., Mukiibi, J., Osei, S., Agbolo, A., Akinode, V., Opoku, B., Olanrewaju, S., Alabi, J., & Muhammad, S., 2022).  Hassana, I. L., & Sanusi, M., (2019) utilised text-to-speech from English to Yoruba as a plugin in Mozilla browser using google API for text-to-text, and a text-to-speech in Yoruba, using online platform application; the collection of the S2ST dataset is a challenge at present (Jia, et al., 2019). Also, the performance of speech-to-speech translation is associated with the volume of the dataset (Julius et al., 2024). Models trained on small datasets give poor performance (McArthur & Richardson, 2023).

In the health domain, public health professionals need a system that can translate their conversation into the community (or population) language in real time. Such a system will reduce the language barrier, save time, save lives, bring out the professionality of their work, and aid the communication of health-related information to the public Javaid, M., Haleem, A., & Singh, R. P., 2023; Panayiotou, A., Hwang, K., Williams, S., Chong, T. W. H., LoGiudice, D., Haralambous, B., Lin, X., Zucchi, E., Mascitti-Meuter, M., Goh, A. M. Y., You, E., & Batchelor, F., 2020; Tu, T., Palepu, A., Schaekermann, M., Saab, K., Freyberg, J., Tanno, R., Wang, A., Li, B., Amin, M., Tomasev, N., Azizi, S., Singhal, K., Cheng, Y., Hou, L., Webson, A., Kulkarni, K., Mahdavi, S. S., Semturs, C., Gottweis, J., Natarajan, V., 2024). The model’s benefit can be in terms of a conversational SoftBot in real time for public use or one-on-one physical conversation between health practitioners and the public. Individuals will not need to search for a transcriber to converse or go a longer distance in search of a public health practitioner who understands and speaks his or her language. Medical speech corpora are not very available to train S2ST models, especially for low-resourced languages like English to Yoruba (Cherednichenko, O., 2020; Grabar, N., & Cardon, R., 2019; Qian, K., Huang, R., Bao, Z., Tan, Y., Zhao, Z., Sun, M., Hu, B., Schuller, B. W., & Yamamoto, Y., 2023). In addition to this, the output audio samples of the target language are not real representations of the input audio samples due to loss of speech features such as the emotion, voice, and prosody of the input audio sample (Limbu, 2020) and the uncertain alignment between the input and output spectrograms is challenging (Jia, et al., 2019). Hence, the speech discretisation (Chang et al., 2023, Hsu, W. N., Bolte, B., Tsai, Y. H. H., Lakhotia, K., Salakhutdinov, R., & Mohamed, A., Rubenstein, et al., 2023) which ensures reduced errors in the training datasets will be utilised for improved transformer architectures in this work. Although, no model had been developed for English-to-Yoruba speech translation use case but it is of great expectation based on findings from literature that feeding the MFCCs features to a discretised model has the benefits of enhancing the model performance.

# 1.3 Aim and Objectives

## 1.3.1 Aim

The aim of this research is to develop an automatic Speech-to-Speech synthesis architecture for the English-to-Yorùbá language pair based on a neural machine translation approach.

## 1.3.2 Objectives

The specific objectives to realise the aim are to:

- create general and public health speech corpus for English-to-Yorùba language pair, focusing on the accent of the speakers of the two languages.

- carry out preprocessing of the the collected data in (i)

- design and implement an improved architecture for speech-to-speech translation (S2ST) of English-to-Yoruba language pair.

- develop a prototype Softbot based on the S2ST architecture in (iii) for malaria public health education use case.

- evaluate the S2ST architecture and the prototype Softbot.

# 1.4 Scope of the Study

The proposed research aims to develop speech-to-speech synthesis for English-to-Yorùbá language translation for engagement between medical practitioners and patients as well as between a public health practitioner and their communities. Both health and general English- Yorùbá speech corpora will be utilised for training the model to be developed. The source and target languages are English and Yorùbá languages respectively. The parallel speech corpora, which involve both general and malaria-related health data, will be pre-processed and fed as input to the improved transformer architecture to be developed. This improved speech translation architecture, consisting of a base model and a transfer learning model, will be utilised for training and testing, and evaluations of these models will be made using metrics such as MOS, BLEU scores, and statistical metrics.  A prototype Softbot based on the S2ST model will be developed and utilised for malaria public health education. Besides, the models could also be used for public engagements for translation purposes from English to Yorùbá between public health practitioners and the population, as well as aiding the translation service between medical practitioners and patients who have, over the years, relied on human language translators.

Hence, this proposed work will be limited to the translational use case of language translation from English to Yorùbá languages leveraging on the existing end-to-end approach without taking into consideration the conventional speech-to-speech translation from English to Yorùbá languages. Besides, the models to be developed will not include the conversational use case of language translation for medical and public domain applications.

# 1.5 Justification for the Study

Speech-to-speech translation can minimize the language barriers experienced by individuals in their conversations with colleagues who speak other languages. Direct speech-to-speech translation to be considered in this work has the benefit of preserving the prosody, emotion, accent, and voice of the speaker, thereby improving the overall performance of the system. The latency associated with the conventional approach will be reduced to the barest minimum. Preserving the speech features ensures that the source speaker’s voice is preserved in the output target language (Yoruba). This also involves training models to learn the voice of the speaker to ensure naturalness and SOTA results. Reducing the delay ensures that the target speech is obtained in real-time without delay. More speech data for English-Yoruba language pairs will be publicly available through this work for more research in this field. Public and medical health practitioners will find their engagements with the public and patients easier than ever, and there will be no impediment in their dissemination of messages. Also, the public will utilise the Softbot for malaria health-related translational tasks and personalized learning.

# 1.6 Expected Contributions to Knowledge

The expected contributions to knowledge in this research work are as follows:

- General and public health corpus for English-to-Yoruba direct S2ST corpus will be created and open-sourced for the language technology research community.

- Improved data preprocessing algorithms for audio corpus

- An improved direct S2ST architecture for English-to-Yoruba that can be fine-tuned for other low-resourced African language pairs.

- A crowd-sourcing web application and a prototype S2ST Softbot could be used for translation within the public health education domain and adapted for other public address translation use cases.

# CHAPTER TWO

# LITERATURE REVIEW

# 2.1 Introduction

This section of the proposal investigates the theories and foundational knowledge in relation to the synthesis of speech from other audio files (languages). The technologies involved, as well as the statistical background information, are highlighted. In addition to this are the reviews of researchers, experts, and professionals in this field and what they have been able to achieve. This considers the findings/results of their experiments and their exploration of the well-known deep learning architectures, transformers, etc, for modelling speech-to-speech synthesis.

# 2.2 Speech Production and Translation in Humans

## 2.2.1 Speech Production and Recognition

A general overview of the speaker model for speech production and speech recognition is given in Figure 2.1, while Figure 2.2 gives more elaborate details of what transpired inside a speech recognition system.

Figure 2.1 comprises a speaker model and speech recognizer for generating speech and text equivalents. The speaker model consists of the speaker's intention and speech production mechanism parts. The first part commences with the development of thought, which is then expressed as a sequence of words, W. To utter the sequence of words as sounds, the speaker then sends appropriate control signals to the articulatory speech organs that form the speech sounds to be utter, which are in proportion to the constructed sentence W. This shows that the speaker model follows three stages: development of thoughts, expression of thought as a sequence of words, W, and then utterances of W as sounds after appropriate triggering of the articulatory speech organs. The speech waveform formed is represented as s[n], as shown in Figure 2.1. The output of this model is fed into the second part, which is the speech recognizer. The first part of the speech recognizer is the acoustic processor, which preprocesses the speech signal and extracts relevant features. X. Then, the feature extracted indicates some phonemes, words, and sentences of the speech signal. This goes into the linguistic decoder that explores the Viterbi algorithm of the Hidden Markov Model (HMM) to choose the maximum sequence of words, W, that relates to the observed feature extracted. Language modelling, dictionary development, and acoustic modelling are also part of the speech recognition system, which are shown in Figure 2.2. From that figure, it can be seen that the Language model, the acoustic model, and the word lexicon are the three models that form the decoding algorithm of the speech recognition system. The acoustic modelling exploring the Viterbi algorithm, or the Baum Welch Expectation Maximization algorithm, produces the maximum sequence of words starting by comparing phonemes generated with word lexicon and language model to get the probability of production of a word to form the intended sequence. The language model is an N-gram, which can be 2 grams, e.g., “He comes,” 3 grams, e.g., “He comes home,” etc. The output of this pattern classification is confidence scoring, which gives the confidence score for each word in the sequence of words (Rabiner, L. R., & Schafer, R. W., 2007). From Figure 2.2, the generated output, an equivalent of the input speech signal, which was constructed from their phonemes, was “Hello World,” with a probability of 0.9 and 0.8, respectively.

Figure 2.1: Conceptual Model of Speech Production and Speech Recognition Processes (Rabiner et al., 2007)

Figure 2.2: Block Diagram of an Overall Speech Recognition System (Rabiner et al., 2007b).

## 2.2.2 Speech Translation in Human

Translating speech from one language to another has been of utmost desire. Humans, at an early age, utilised human translators to achieve these tasks. Speech produced from the vocal tract of an individual, as illustrated in Figure 2.1, is at some point translated to other languages based on the individual involved in the conversation. Humans have been able to achieve this with some outstanding results where the produced speech is converted from one language to another. However, only a few trained human translators can translate from one language to others across different national or multi-national languages (Kocmi, Tom., Macháček, Dominik., & Bojar, O., 2022; Workshop, B., :, Scao, T. Le, Fan, A., Akiki, C., Pavlick, E., Ilić, S., Hesslow, D., Castagné, R., Luccioni, A. S., Yvon, F., Gallé, M., Tow, J., Rush, A. M., Biderman, S., Webson, A., Ammanamanchi, P. S., Wang, T., Sagot, B., … Wolf, T., 2022). Hence, there is a need to develop speech technology that could convert from one language to another.

## 2.2.3 Low and High-Resourced Language

The language itself plays a vital role in the language translation. Some languages are well-known and spoken by a large population, while others are yet to gain popularity or are spoken by a minority of the world population (Bozkurt, A. ;, Junhong, X. ;, Lambert, S. ;, Pazurek, A. ;, Crompton, H. ;, Koseoglu, S. ;, Farrow, R. ;, Bond, M. ;, Nerantzi, C. ;, Honeychurch, S. ;, Bali, M. ;, Dron, J. ;, Mir, K. ;, Stewart, B. ;, Costello, E. ;, Mason, J. ;, Stracke, C. M. ;, Romero-Hall, Bozkurt, A., … Stracke, J., 2023). These languages could be classified as low-resourced or high-resourced languages.

### 2.2.3.1 Low Resourced Language

These are languages whose parallel speech pair datasets are not mostly available for language translation modelling and training. Data insufficiency has led to little development of language translation for these languages (Ranathunga et al., 2021). Parallel pair corpus of both source and target language are mostly available for well-known European nations and American or high-resourced languages. Most of the African indigenous languages, including Yorùbá, have lesser research works done in this field of language translation (Gutkin, A., Demirsahin, I., Kjartansson, O., Rivera, C. E., & Túbòsún, K., 2020).

### 2.3.3.2 High-Resourced Language

These are languages whose corporal are publicly available and in large amounts for language translation modelling and have seen great development in this domain. Such languages include English, Chinese, French, Russian, etc. However, most of these languages also have challenges with enough parallel pairs of training data for modelling language translation, where the expertise can synthetically generate the source and/or target speech corpus (Barrault et al., 2023).

# 2.3 Overview of Automatic Speech-to-Speech Translation

## 2.3.1 Traditional Speech-to-Speech Translation

In the traditional approach to speech-to-speech translation, various statistical techniques like the HMM, Viterbi algorithms, and Balm welch expectation maximisation were utilised to model the speech-to-speech translation. During this era, statistical approaches were utilised to model the ASR, MT, and TTS. For the ASR, for instance, the algorithms were utilised to model the conversion from speech to text, where the acoustic model was done using the GMM, and the decoding search of the correct sequence of texts was carried out using the Viterbi algorithm with the language models, and lexical model. This output is fed to a statistically modelled MT where the target language text is obtained. The output of this MT is fed to a statically modelled TTS, where the output speech of the target language is obtained. Other approaches utilised during this era are the rule-based approach for MT, concatenative approaches for TTS, etc.

### 2.3.1.1 Probabilistic Approach to modelling ASR

The probabilistic approach to ASR is mostly referred to as a pipeline, as shown in Figure 2.3 because it is a system of large components where the output of one stage is the input to the next stage of the system.

Figure 2.3: Pipeline (Probabilistic Approach) ASR (Medeiros, E. F., 2023a)

The preprocessing of the audio signals is carried out first, then the next stage is the framing and windowing to minimize the discontinuity of the signal (Armaisya, D. D., Pamungkasari, P. D., Rifai, A. P., Sholihati, I. D., & Sakarkar, G., 2025) . Through this, the audio signal is split into slices typically of range 20 ms to 40 ms with a well-known standard value of 25 ms. Next is the smoothening of its border to minimize the impact of framing (Medeiros, 2023). The next step is the application of different filters, as shown in Figure 2.4, to extract the feature vector coefficients, which in this case are the Mel Frequency Cepstral Coefficients (MFCC).

Figure 2.4: Different Speech Features Obtained at Different Processing Stages (Medeiros, 2023).

From Figure 2.4, the framing output goes into the FFT filter to change the signal to the frequency domain from the time domain for easy analysis and visualization, and this is converted to a logarithm scale for ease of understanding and differentiation among sounds of the speech signal. The extracted feature vector is trained using the GMM, and a beam search for the correct sequence of words replica of the speech input is decoded using the Viterbi algorithms.

### 2.3.1.2 Statistical Approach to Machine Translation

The statistical approach for modelling MT can be carried out by adjusting the Bayes equation (2.1) implemented for modelling ASR where the acoustic model, PA(X/W) is being replaced by the translation model and the language model, PL(W) replaced by the target language model.

P(W/X)=W'= agrmaxWPA(X/W)PL(W)                                                                               (2.1)

Hence, the MT translation model is given in equation (2.2) (de Martos, A. M., 2022) as

P(y/x)=y'= argmaxyϵ YPT(x/y)PL(y)                                                                                        (2.2)

Where:

PT(x/y) = Translation model

PL(y) = Target language model

P(y/x)=argmaxyϵ Y() = The decoded word string that corresponds to the source word

string and can be interpreted as the probability of y being the translation of x.

The translation model is trained on source-target parallel sentences so that the mapping of the source to target sentences is carried out. The language model is trained on the single language data of the needed target language such that the correct word sequence (sentence) equivalent of the source sentence is decoded.  The HMM utilising the Viterbi algorithm is one of the algorithms for searching the correct word string to be outputted, which is the most likely string equivalent of the source string.

### 2.3.1.3 Concatenative Speech Synthesis

The CSS approach to modelling TTS involves concatenating several pieces of speech, otherwise called units (generated from texts), to form the decoded speech waveform. For this approach, there is the need for a large recording database (Baral, 2025; Nallabala, N. K., Souprayen, B., Ramasamy, M., Penumarti, S. K., & Navuluri, N. C., 2025; Suikkila, 2025)  that stores and covers all possible combinations of these speech units, which are merged to construct the output speech utterance, and a unit selection algorithm such as Viterbi decoding (White & Watson, 2025)  using prosodic mismatch objective cost measures and spectral discontinuity instead of HMM, is utilised for these units’ selection (de Martos, 2022).

### 2.3.1.4 Statistical Parametric Speech Synthesis

In this approach to generating speech from text, the speech generated relies on a statistical approach rather than depending on the pre-recorded speech. Trained on text-audio pair similar to ASR, it consists of three stages: text-analysis, acoustic model or parametric prediction model, and vocoder (de Martos, 2022). The recording is mostly carried out in professional studio settings to ensure good audio quality and the best acoustic modelling (de Martos, 2022).

## 2.3.2 Conventional Speech-to-Speech Translation

The conventional approach to modelling speech-to-speech synthesis involves the cascaded blocks of SR, MT, and TTS. The three blocks are arranged in a pipeline where the output of one is the input to the next block. One of the problems associated with this approach is the delay experienced from one block to the next, leading to an overall delay in the system (de Martos, 2022). Another issue with this approach is the misalignment between the input and output, which leads to the generation of errors within the system. Both the statistical and the neural network approaches are utilised to model each of the blocks of the speech-to-speech translation model. The pipeline for the conventional speech-to-speech approach is illustrated in Figure 2.5.

Figure 2.5: Conventional Approach to Modelling Speech-to-Speech Translation

### 2.3.2.1 Automatic Speech Recognition

Speech recognition, also known as Automatic Speech Recognition or computer speech recognition and speech-to-text conversion (Desai, N., Dhameliya, K., & Desai, V., 2013), is the ability of a program or machine to recognise or identify the phonemes, words, phrases, or sentences of a speaker's spoken words (Armaisya et al., 2025; Medeiros, 2023). This means that the Automatic Speech Recognition (ASR) will take speech as its input, process or decode the speech (based on the training it has acquired), and produce the text equivalent of the same spoken words or the textual translation of speech signal (O’Shaughnessy, D., 2008). Other useful outputs from the ASR include the speaker's emotional state, the speech-language, and the identity of the speaker (Ververidis, D., & Kotropoulos, C., 2006). The speech signal, whether the original human voice signal or the recorded audio signal, represents the human vocal tract. The human speech signal has a bandwidth of within 0 to 20KHz, which is wide enough to allow good quality speech signal to be obtained, unlike the audio signal that is bandwidth limited as in the case of AM and FM (Ververidis & Kotropoulos, 2006).

The conversion of speech to text can be achieved via ASR and other technologies like HMM and Artificial Neural Networks (ANNs). The goal of an ASR is to efficiently and accurately obtain the written texts from a speech signal, which does depend on the acoustic environment of the speaker (which can be outdoors, quiet office, noisy room), the speaker's accent, and device's type used in recording the speech (which can be a microphone or transducer). The biggest challenge of an ASR system is its ability to handle its variabilities, such as the speaker’s environments, the types of speakers such as dependent speakers, independent speakers, adaptive speakers, variation in environments, etc. (O’Shaughnessy, 2008). The most difficult variability that ASR must handle are the channel noise, background noise, and other external distortions. Hence, an ASR must be designed to meet and cater to this challenge (O’Shaughnessy, 2008). However, its ultimate goal is to perform as a human listener, which is yet to be achieved (Chang, X., Watanabe, S., Delcroix, M., Ochiai, T., Zhang, W., & Qian, Y., 2025; Rabiner et al., 2007). Vocal sound (single utterance or separate utterance), vocabulary (to determine the system complexity, precision, and performance), and speaker (independent or individual speaker) are the parameters used to classify speech recognition (Trivedi et al., 2018) while speech itself can be classified as highly conversational speech (meetings and discussion of involving group of people), isolated speech (containing pauses between words), spontaneous speech (human to human dialog), continuous speech (without pauses between words), and spelled speech (having pauses between letters or phonemes) (Medeiros, 2023).

### 2.3.2.2 Machine Translation

This is a technique implemented to translate text in one language to text in another language, which is otherwise called the target language. It is a type of translation that shares similarities with speech-to-speech translation. Unlike speech-to-speech translation, which involves speech conversion, it involves text conversion. The use of MT started in the 1940s and is seen as a threat to professional human translators even though it is technologically indispensable and economically viable (Hutchins, J., 1997). Some well-known MT models are Li, R., Wang, X., & Yu, H. (2020); Rapp, R., (2021); and Xue, L., Constant, N., Roberts, A., Kale, M., Al-Rfou, R., Siddhant, A., Barua, A., & Raffel, C., (2020). Mathematically, MT translates source sentences, x into target sentence into target sentence, y given in equation (2.3) (de Martos, 2022) as

x= x1, x2, x3,…, xi   xi∈ X

y= y1, y2, y3,…, yi   yi∈ Y                                                                                                       (2.3)

Where:

x and y are the source and target sentences.

xi and yi are the source and target words in each sentence and;

X and Y are the vocabularies that contain the source and target words.

The approaches to modelling the MT are the rule-based approach, statistical approach, and neural network approaches. Among these approaches, the neural network approach has yielded state-of-the-art results.

### 2.3.2.3 Text-To-Speech

This does the reverse of the speech recognition. It takes text as input and outputs the speech equivalent of the text. It is a speech synthesis system that generates utterances of the texts given as input. The TTS consists of text analysis, natural language processing, and digital signal processing (Yao & Poonpon, 2025) . The input to the TTS is the sequence of texts y1, y2, y3,…, yi where the initial output is the speech Mel-spectrogram features x1, x2, x3,…, xi which is further processed to obtain the speech audio waveform. In comparison to speech recognition, where the probability of word string given feature vector, P(W|x) is decoded as the output, in TTS, the probability of the feature vector given the word string PxW=P(x|y) is decoded where the word string, W has been replaced by a string y.

## 2.3.3 Neural Speech-to-Speech Translation

This involves using a deep neural network to model speech-to-speech synthesis. It works by training on a large dataset of speech of both the source and target language to model speech-to-speech synthesis. Machine learning models such as the RNN, LSTM, and BiLSTM have been implemented to achieve the translation of speech from one language to another. The BiLSTM has found application in sequence-to-sequence learning for converting sequences from one language to another language (Limbu, 2020). The RNN suffers from dependence issues, as it cannot go deeper into the network to make predictions of the present output. Hence, the LSTM was developed to overcome this, but it was deficient in fulfilling its task when a large volume of parallel speech corpus was involved. This led to the utilisation of the transformer architecture. The result of this approach is a high-quality output. However, as the size of the dataset increases, the performance of the system is reduced; hence, there is a need to develop techniques to overcome this drawback. At present, the transformer architectures are implemented to overcome the deficiency of the neural network approach in a sequence-to-sequence fashion.

### 2.3.3.1 Neural Network Approach to Automatic Speech Recognition

Recent technological advances have seen the application of ANN in Natural language processing, ASR, computer vision, handwritten recognition, etc. This development, coupled with the large amount of available data for training, has increased the exploration of ANN in these areas of research.

A convolution neural network is an example of a neural network modelling that explores the convolution property by convolving many layers within the network. It has found applications in computer vision for filtering images. The same is true in its application to ASR, where different filters represented by the layers of the network are used to extract relevant features from the input audio signal, unlike the probabilistic approach that exploits framing, windowing, spectral, and cepstral analysis for the same function. Wav2Letter is an example of an ASR system that uses end-to-end CNNs, where graphemes, which are base unit representations, are used instead of phonemes. A grapheme is a single letter that represents a phoneme; hence, there is no need to use phonetic transcription (Medeiros, 2023). Exploring the Wav2Letter approach, NVIDIA developed a new ASR system called Jasper that uses 1 Dimensional Convolutional Neural Networks (1D-CNN) (Li, J., Lavrukhin, V., Ginsburg, B., Leary, R., Kuchaiev, O., Cohen, J. M., Nguyen, H., & Gadde, R. T., 2019a; Medeiros, 2023). It has 54 convolutional layers (333M parameters), while the smaller model uses 34 (201M parameters) and a mix of activation and normalization functions such as the batch normalization and ReLU functions, and it has a tremendous improvement on the WER of the ASR system. Jasper belongs to the end-to-end ASR models that replace the pronunciation model (language model and phonetic model) and the acoustic model with CNN. It uses Mel filterbank feature coefficients that are calculated from 20 ms windows with an overlap of 10 ms in which 64 features were calculated. The output probability distribution is in characters per frame. Jasper has B*R block architectures where each B block has R sub-blocks (with the same number of output channels) that apply the 1D Convolution, ReLU function, batch normalization function, and dropout. Other types of normalization functions, such as the layer and weighted normalization; activation functions, such as clipped ReLU and leaky ReLU; and gated units, like the gated linear unit (GLU) and gated activation unit (GAU), were explored in Jasper. Besides, the modelling of the language model was done using Recurrent Neural Network (RNN), ConvLM, N-gram language model, Long Short-Term Memory (LSTM), as well as a Transformer XL. It also has different architectures for the residual connections of layers within the network, such as Residual, Dense Residual, DenseNet, and DenseRNet (Li J., et al., 2019a).

### 2.3.3.2 Neural Network Approach to Machine Translation

This is also termed Neural Machine Translation (NMT) approach to modelling MT. It is sequence-to-sequence modelling that is based on transformer architecture implemented at the encoder and decoder. It has been implemented to achieve improved MT performance and SOTA results (de Martos, 2022). It models MT by directly estimating the probability of the output sentence given the input word string as given in equation (2.4) (Kalchbrenner, N., & Blunsom, P., 2013; Sutskever, I., Vinyals, O., & Le, Q. V., 2014).

py|x= j=1Jp(yj|y1j-1, x)                                                                                                       (2.4)

The beam or greedy search decoding algorithm is implemented to output the correct word sentence that comes from the source sentence. Other approaches to modelling MT involve standard neural networks like CNN and RNN, but the results achieved can not be compared to SOTA results obtained using transformer architectures (Wu, Y., Schuster, M., Chen, Z., Le, Q. V, Norouzi, M., Macherey, W., Krikun, M., Cao, Y., Gao, Q., Macherey, K., & others., 2016).

### 2.3.3.3 Neural Network Approach to Modelling TTS

DNN has been utilized to model TTS using SPSS by replacing some HMM components (Fan, Y., Qian, Y., Xie, F.-L., & Soong, F. K., 2014). In the late 2010s, with the introduction of an end-to-end autoregressive neural vocoder and acoustic model, the quality and naturalness of the generated speech were improved. The end-to-end model works by taking texts or phonemes as input to an encoder and generates the output speech Mel-spectrogram by a decoder, after which a vocoder generates the speech utterance. Some of the pioneering TTS models that have gained popularity are Deep Voice (Ping, W., Peng, K., Gibiansky, A., Arik, S. O., Kannan, A., Narang, S., Raiman, J., & Miller, J., 2017), WaveNet (Oord, A. van den, Dieleman, S., Zen, H., Simonyan, K., Vinyals, O., Graves, A., Kalchbrenner, N., Senior, A., & Kavukcuoglu, K., 2016), and Tacotron (Shen, J., Pang, R., Weiss, R. J., Schuster, M., Jaitly, N., Yang, Z., Chen, Z., Zhang, Y., Wang, Y., Skerrv-Ryan, R., & others., 2018). A neural TTS model is illustrated in Figure 2.6

Figure 2.6: A Generic Two-Stage Neural TTS Architecture (de Martos, 2022)

As illustrated in Figure 2.6, the neural end-to-end TTS synthesis consists of two stages. The first stage is the end-to-end modelling of the input sentence to generate the output spectrogram features. The input sequence, which is converted into the vector (using embedding) that is easily understood during training, is fed to an encoder; the encoded output vector is passed through an attention mechanism to ensure all features of the text input are well extracted to ensure accurate mapping of texts to spectrogram features. The output of this is fed to a decoder that finally generates the output spectrogram. The encoder and decoder are nothing but stacked of neural network algorithms such as MLP, LSTM, BiLSTM, RNN, etc. The second stage converts the acoustic speech features to output waveform using Vocoder (Shen et al., 2018) or another algorithm (Wang, Y., Skerry-Ryan, R. J., Stanton, D., Wu, Y., Weiss, R. J., Jaitly, N., Yang, Z., Xiao, Y., Chen, Z., Bengio, S., & others., 2017).

For the first stage, which is text-to-spectrogram, the acoustic model is trained on a collection of text-audio pairs represented in equation (2.5) as:

D={xi, wi}i=1N  <text, audio>                                                                                          (2.5)

And its goal is to minimise the loss between the predicted, y and target or ground truth spectrogram, y obtained from the audio waveform, w (de Martos, 2022). According to Murphy, (2022), a teacher forcing algorithm is utilised to train the autoregressive model in which the decoder output at time, t is conditioned by the ground truth spectrogram frame, yt-1. Hence, an exposure bias (Liu, R., Sisman, B., Li, J., Bao, F., Gao, G., & Li, H., 2020), which is a mismatch between inference conditions and training, is introduced.

For the second stage, which involves the spectrogram to audio waveform conversion, the vocoder is trained on a collection of un-transcribed audio given in equation (2.6) as:

D={wi}i=1N                                                                                                                             (2.6)

From the training of equation (2.6), an intermediate representation of the spectrogram of the audio waveform can be obtained.

For the last 5 years, other TTS models that have achieved enhanced performance in terms of improvement of audio quality through Vocoder and matching the performance of WaveNet for TTS modelling are: FFNET Jin, Z., Finkelstein, A., Mysore, G. J., & Lu, J., (2018), WaveRNN Kalchbrenner, N., Elsen, E., Simonyan, K., Noury, S., Casagrande, N., Lockhart, E., Stimberg, F., Oord, A., Dieleman, S., & Kavukcuoglu, K., (2018), Parallel WaveNet Oord, A., Li, Y., Babuschkin, I., Simonyan, K., Vinyals, O., Kavukcuoglu, K., Driessche, G., Lockhart, E., Cobo, L., Stimberg, F., & others, (2018), WaveGlow Prenger, R., Valle, R., & Catanzaro, B., (2019), LPC-Net Valin, J.-M., & Skoglund, J., (2019), as well generative approach such as the MelGAN Kumar, K., Kumar, R., De Boissiere, T., Gestin, L., Teoh, W. Z., Sotelo, J., De Brebisson, A., Bengio, Y., & Courville, A. C., (2019), Parallel WaveGANs Yamamoto, R., Song, E., & Kim, J.-M., (2020), HiFi-GAN Kong, J., Kim, J., & Bae, J., (2020) that tend to increase the speed and computational cost.

## 2.3.4 Direct Speech-to-Speech Translation

This is also called an end-to-end speech-to-speech translation, where the deep neural network is utilised to obtain state-of-the-art results. In this method, the input side contains an encoder that consists of a stack of RNNs, BiLSTMs, or LSTMs networks, as well as the output encoder. There is also a transformer architecture with state-of-the-art techniques like positional encoding, embeddings, attention mechanism, etc., to obtain SOTA results. The input to such a model is sequential data, and the output generated is also sequential data.

### 2.3.4.1 Generic Architecture

The generic direct S2ST architecture is illustrated in Figure 2.7 (Jia, Y., Ramanovich, M. T., Remez, T., & Pomerantz, R., 2022). As shown in Figure 2.7, the components of the architecture are the spectrogram, synthesizer, blocks of encoder, attention mechanism, and decoder; also utilized is the Graffin Lim Vocoder, and the outputs are the final speech waveform (English) and English phoneme. The constituent blocks of the architecture are explained in subsequent sub-sections. The architecture was an improvement to the original Translatotron that was carried out in (Jia, et al., 2019).

Figure 2.7: Generic Direct Speech-to-Speech Architecture (Translatotron 2) (Jia et al., 2022)

### 2.3.4.2 Speech Mel Spectrogram

This is a two-dimensional plot with time on the abscissa and frequency on the ordinate, along with the amplitude as a scale of color saturation or a colored heat map. It is a graph that shows the relationship between the spectrum of frequencies and time, where the amplitudes, which are the heat maps, are representations of audio files at a particular frequency and time. For using the S2ST approach for language translation, the model is inputted with the spectrograms of both the sourced and target languages during training. During evaluation and testing, only the spectrogram of the source language is fed as input to generate the output language spectrogram, which is later changed to an audio file via block such as vocoder as was done (Jia, et al., 2019). Displayed in Figures 2.8 and 2.9 are the speech spectrograms of the source English audio (“I will be back soon”)-target Swedish audio (“Jag Kommer Tillbaka Snart”) pair.

Figure 2.8: Spectrogram for English Audio - "I will be back soon” (Limbu, 2020)

Figure 2.9: Spectrogram for Swedish Audio - "Jag Kommer Tillbaka Snart" (Limbu, 2020)

### 2.3.4.3 The Fast Fourier Transform

The FFT is a faster algorithm for computing the DFT of a discrete-time signal. The speech spectrogram is used to gain an understanding of how the phonetic information is encoded in the speech signal (Rabiner & Schafer, 2007). Currently, spectrograms are made using digital speech processing (DSP) techniques and are displayed as grey-scale images or pseudo-color on computer screens (Rabiner & Schafer, 2007). The speech spectrograms are simply a display of the plot (time on x axis against frequency on the y axis with vertical color bar to the right of the graph) of the log of the Short Time Fourier Transform (STFT) given in equation (2.7) and plotted in Figure 2.10, where analysis windows of different lengths were used for the two plots and the speech waveform “Should we chase” is plotted atop them.

Str, fk=20log10XrRk=20log10|XrRk|                                                                   (2.7)

where:

tr=time in seconds

fk=frequency in Hz

XrRk=STFT, which is the DTFT of the input signal xn[m]=xmwn-m

xmwn-m=amplitude-weighted by the sliding window, wn-m

XrRk=has the interpretation of N Point Discrete Fourier Transform (DFT)

tr=rRT and fk= KNT

T=Samling period of the discrete-time signal xn= xa[nT]

XrRk= m=rR-L+1rRxmwrR-me-j2πkmN     k=0, 1, …, N-1,                       (2.8)

XrRk= m=0L-1xrR-mw[m]ej2πkmN    k=0, 1, …, N-1                                    (2.9)

And

XrRk= XrRke-j2πkrRN    k=0, 1, …, N-1                                                           (2.10)

N=Total number of uniformly spaced frequencies within 0 ≤ ω ≤2π   N-point

Length of the DFT

ω=angular frequency in radian per seconds

L=Window length in samples

wm is within the range 0 ≤ m ≤L-1  (Rabiner & Schafer, 2007)

Equation (2.8) shows that the STFT is computed in the frequency domain in a finite-duration window moved in steps of R > 1 samples in time.

When the window length chosen is short, the resulting spectrogram is called a wide-band spectrogram, but when the window length is long, it is called a narrow-band spectrogram (Rabiner & Schafer, 2007).

Figure 2.10: Spectrogram for Speech Signal “Should we chase” with Hamming Window Length M = 101 for the Upper Plot and M = 401 for the Lower Plot (Rabiner & Schafer, 2007)

### 2.3.4.4 Synthesizer

The synthesizer in the Translatotron 2 was built using upsampler, duration predictor, mixture of RNN (LSTM), and Convolutional layer. The inputs to the block are outputs from the attention mechanism and phoneme decoder. The function of the synthesizer is to synthesize the needed output spectrogram (English spectrogram). The upsampler increases the dimension of the duration predicted inputs samples into the block after which an autoregressive decoding is carried out by the LSTMs – which is the main decoder of the synthesizer; and further processing carried out by the Convolutional layer.

### 2.3.4.5 Encoder

This consists of a stack of deep learning architectures. They could be LSTM, BiLSTM, MLP, etc. The outputs of the encoder are vector representations of the input Mel spectrogram. For the speaker encoder, this is a trained model that is used to preserve the features of the speaker. Features such as emotion, voice, and prosody are transmitted to the output audio collected by the speaker encoder. It can be trained for a single-speaker voice or more than a speaker's voice. The concatenation block concatenates the output of both the encoder and speaker encoder.

### 2.3.4.6 Attention Mechanism

This is the mechanism to know how much a particular input sequence or utterance is contributing to the prediction of the audio samples. It involves the generation of another vector that is able to retain the features of the input layers in the hidden layer and carry out proper mapping of the source and target inputs during the training phase. Hence, it makes the model “attend to” certain parts of the input sequence to output the audio file. It enhances the overall output audio waves of the model.

### 2.3.4.7 Decoder

This could also be stacked of deep learning algorithms like the case of the encoder to generate the output spectrogram taking the output of the attention mechanism as its input. The decoder outputs the phonemes of the target language (English). The output of the decoder could be fed to a dense layer (original Translatotron) (Limbu, 2020), which is a neural network architecture that carries out further processing on the output spectrogram. Here, the general modelling equation of a neural network is illustrated in equation (2.11).

fz=activation(w.x+b)                                                                                                   (2.11)

Where:

fz=output of the dense layer to be fed as input to a vocoder block

w=weight of the last deep learning algorithm e.g. LSTM of the decoder

x=input, which can be chosen to be input tensor (Limbu, 2020)

### 2.3.4.8 Vocoder

The output spectrogram obtained from the synthesizer is fed to a vocoder, which performs the the conversion of the spectrogram to the expected English waveform, which is a replica of the transmitted audio Spanish waveform. This could be WaveRNN or Graffin Lim Vocoder Griffin, D., & Lim, J., 1984; Jia, et al., 2019). For better enhancement and generalisation of the English speech synthesized, Translatotron 2 combined the decoding tasks of both the phoneme and spectrogram during the same training time, unlike in the original Translatotron (Jia, et al., 2019), where phoneme decoding training tasks of both English and Spanish were separated from that of the English audio waveform decoding.

# 2.4 State-of-the-Art Approaches for Speech-to-Speech Translation

## 2.4.1 Sequence-to-Sequence Learning

This refers to a technique where the input and output are variable-length sequences. It is a deep learning approach that involves training on sequential data. For example, in the modelling of ASR, the input is the sequence of acoustic features while the output is a sequence of words, and the reverse is the case for TTS; for MT, the input and output are sequences of words (de Martos, 2022). It is shortened as Seq2Seq modelling. In this type of learning, an encoder and decoder are implemented on the fixed length of input source data to generate a fixed-length output target. The input and output are mapped to each other (Limbu, 2020). The encoder and decoder, in most cases, could be MLP (Radford et al., 2023), BiLSTM, or LSTM (Jia, et al., 2019). The use of sequence-to-sequence learning has yielded great performance for language translation for both texts and audio.

## 2.4.2 Encoder-Decoder Network

This refers to the integration of encoder and decoder in a sequence-to-sequence learning fashion to achieve the intended task. It has found applications for both text and audio translation from one language to another language (Limbu, 2020). The encoder could be networks of MLP, LSTM, or BiLSTM, which is also the case for the decoder. A typical example of an encoder-decoder network is displayed in Figure 2.11. The encoder consists of a stacked of 3 BiLSTM while the decoder consists of a stacked of 2 LSTM networks.

Figure 2.11: A Basic Encoder-Decoder Network using LSTM (Limbu, 2020)

As shown in Figure 2.11, each recurrent unit in the encoder accepts the input at each timestep and passes it through the hidden layer to the next unit. The final output passes to the decoder, where it is decoded at the corresponding time step to generate the final outputs. The encoder-decoder architecture is one of the most important aspects of Google’s translation services (Limbu, 2020). For ASR, the decoder is to generate the text representations of the vectors generated from the output of the encoder. Hence, it can be termed a language model (de Martos, 2022); for MT, the encoder and decoder can be taken as language models due to the text involvement of both blocks.

The formula to compute the hidden state from the encoder, the hidden state of the decoder, and the final output are given in equations (2.12), (2.13), and (2.14), respectively (Limbu, 2020).

ht=f(Whhht-1+ Whxxt)                                                                                                   (2.12)

ht=f(Whhht-1)                                                                                                                    (2.13)

yt=Softmax(Wsht)                                                                                                            (2.14)

Where:

ht=current hidden state

ht-1=Previous hidden state

xt=Present input

Whh=weight between two hidden layers*

Whx=weight between a hidden layers and an input*

## 2.4.3 Attention Mechanism

This is one of the important techniques utilized for encoder-decoder networks to obtain enhanced performance of the model trained. It could be self-attention, cross-attention (Radford et al., 2023), multi-head attention (Jia, et al., 2019), etc. During training, there is the need for the final hidden layer to capture all the important features of the input sequence for the proper working of the algorithms, but this becomes a problem when the length of the input sequence is large, as it causes a misalignment between the source and target inputs. To solve this misalignment, the attention mechanism that introduces a new vector (different from the vector generated by the encoder when it acts on the input) is used. This vector preserves all the information in the hidden state of the encoder and aligns them with the target for enhanced performance. Hence, the model can “attend to” particular parts of the input sequence while the input is being mapped to the target, and then it enhances the relationship between the source and target inputs.

## 2.4.4 Sequence-to-Sequence Modelling

As depicted in Figure 2.12, the input sequence, x=[x1, x2, x3,…,xN] of length, Nis being processed by the encoder to generate an output vector, z=[z1, z2, z3,…,zN], of length, N, that is otherwise called the encoder output or encoder hidden state (de Martos, 2022). This output vector is then changed to the output sequence, y=[y1, y2, y3,…,yT] by the decoder, as illustrated in equation (2.15) (de Martos, 2022).

yt=dec(y1t-1, zn)                                                                                                                 (2.15)

Figure 2.12: A Basic Encoder-Decoder Network Showing the Encoded Vectors (de Martos, 2022)

The fixed output vector, zn holds all the relevant features of the input sequence, which can be many words. As initially stated in section 2.4.3, this becomes a problem when the length of the input sequence is long, which depicts the above architecture suffers from memory because it will be difficult to map the input to the target output properly. Hence, a solution to this is the integration of the attention mechanism for Seq2Seq modelling.

For the attention mechanism, a context vector ct given in equation (2.16), which replaces the fixed vector, zn is computed dynamically at each decoding step, and it is utilised.

ct= i=1Nαt,  izi                                                                                                                        (2.16)

Where:

αt,  i = weights assigned to each hidden state of the encoder at the time step, t where the

highest weight is assigned to the most relevant vector, and they are computed by

utilising Softmax function (2.17) as

αt,  i= exp⁡(et,  i)k=1N(et,  k)                                                                                  (2.17)

And its value is within the range 0 ≤αt,  i ≤1, for all i with probability condition:

i=1Nαt,  i=1

Where:

et, i = attention score (energy value) obtained using some scoring function.

The scoring function is a depiction of the relevance of the encoder vector, zi at each time step. For recurrent Seq2Seq models, it is mostly obtained using the previous hidden state of the decoder as given in equation (2.18)

et, i=a(ht-1, zi)  for all i                                                                                                        (2.18)

Where:

a(.) = scoring function (de Martos, 2022)

A better illustration of the attention mechanism is given in Figure 2.13.

Figure 2.13: Attention Mechanism Overview (Zhang, A., Lipton, Z. C., Li, M., & Smola, A. J., 2021)

As illustrated in Figure 2.13, the attention mechanism can be compared to a dictionary look-up that consists of a key-value pair, k1, v1,  k2, v2,…, kN, vN,  of length N, where a query, q ∈Rdq is compared to each key, ki ∈Rdk of the dictionary, and then retrieve the corresponding value vi ∈Rdv. As shown in Figure 2.9, a complex combination of values is obtained for each output. The scoring function, a(.) can be obtained using scaled dot-product (Luong, M.-T., Pham, H., & Manning, C. D., 2015) or additive (Bahdanau, D., Cho, K., & Bengio, Y., 2014) scoring functions as given in equations (2.19) Vaswani, A., Shazeer, N., Parmar, N., Uszkoreit, J., Jones, L., Gomez, A. N., Kaiser, Ł., & Polosukhin, I., (2017) and (2.20), respectively.

aq, k, v= softmaxqTkdv ∈ R                                                                                       (2.19)

aq, k= wvTtanhWqq+ Wkk∈ R                                                                                  (2.20)

Where:

wv ∈Rh, Wq ∈Rh x q, and Wk ∈Rh x k are learning model’s parameters (de Martos, 2022)

For the scaled dot-product equation (2.19), q, and k have the same dimension, d (de Martos, 2022)

## 2.4.5 Transformer

This is a type of Sequence-to-Sequence modelling utilising an encoder and decoder at the input and output points. It has found applications for modelling systems that take sequential data as input. Examples of such are speech processing applications such as TTS and ASR, coupled with NLP tasks such as the MT. Some state-of-the-art (SOTA) speech resources (Chang et al., 2023), such as Jasper (Li et al., 2019a), Whisper ASR (Radford et al., 2023), and language translators, have implemented transformer architecture.  It has been implemented for the development of ChatGPT (Haluza, D., & Jungwirth, D., 2023). A typical transformer architecture is shown in Figure 2.14.

Figure 2.14: The Original Transformer Architecture (Vaswani et al., 2017)

As shown in Figure 2.14, the decoding of outputs from the decoder of the transformer encoder-decoder structure does not depend on the deep learning algorithm chosen; rather, it depends on three concepts, which are positional encoding, self-attention, and multi-head attention (de Martos, 2022). It utilises positional encoding and self-attention to investigate other positions of the sequence when it is processing a particular input, while multi-head attention is utilised for improved performance of the model (Murphy, K. P., 2022). The key parts of Figure 2.14 are highlighted below:

### 2.4.5.1 Self-Attention

This is a type of attention in which the encoder attends to itself in computing context-aware encodings whose length is the same as the input sequence. In this case, each input attends to all positions of the input sequence (de Martos, 2022).

Given an input sequence, x=[x1, x2, x3,…,xn] in which xi∈Rd, to compute output sequence, self-attention utilises xi as a query on the key and value of the dictionary that are each depicted by [x1, x2, x3,…,xn].

### 2.4.5.2 Multi-Head Attention

In this type of attention, the attention mechanism can utilize varying representation sub-spaces of queries, keys, and values-this is done to capture different dependencies, such as long-range, short-range, etc., of the input sequence. Each attention head, hi in a multi-head, attention is computed using equation (2.21):

hi= f(Wiqq, Wikk, Wivv)                                                                                              (2.21)

Where:

q=query, where q ∈Rdq

k=key, where k ∈Rdk

v=value, where v ∈Rdv

Wi*=learnable parameters

f.=Attention funct which can be scaled dot-product or additive function.

The multi-head attention head, h which is the output obtained is the scaled value of the vector head (by the learned weight parameters) of size n as given in equation (2.22) as:

h=W0 h1⋮hn                                                                                                                           (2.22)

### 2.4.5.3 Positional Encoding

This is a mechanism that is implemented in a transformer to keep track of the position of each word in an input embedding sequence. The positional encoding is done using sine and cosine functions on the input sequence as illustrated in equation (2.23):

pi, 2j=siniC2jd,  pi, 2j+1=cosiC2jd                                                                                (2.23)

Where:

C = fixed maximum length of a sequence.

j = dimension index of the embedding.

i = position.

d = total dimensions number.

The benefits of this type of embedding are that it allows the model to attend relative positions effortlessly and it allows computation for arbitrary input lengths up to C.

## 2.4.6 Cross lingual Voice Cloning

Voice cloning refers to the synthetic production of a speaker's voice or intonation of one language in another language. For instance, an English speaker's voice is reproduced in the Yoruba language while the accent or intonation is preserved, and it is the English speaker that is being heard speaking Yoruba. For cascaded S2ST, this can be achieved by utilizing Tacotron 2 (for monolingual single speakers), which can be adapted to support voice cloning for language pairs of seen and unseen speakers as well as multiple speakers and languages. In this situation, the TTS system (Tacotron 2) is trained with speaker embeddings and language-specific phoneme embeddings, and then the output waveform or speech is a replica of the speaker's voice (de Martos, 2022). The trained Tacotron 2 is merged with the existing systems of ASR+MT. In direct S2ST, a speaker encoder is utilized (Jia, et al., 2019), where the model is trained with a speaker encoder pre-trained on multiple speakers, and the output speeches are replicas of the speakers’ voices, the model is trained with.

# 2.5 Stages Involved in Neural End-To-End Approaches to Speech Translation

## 2.5.1 Data Collection

The mostly available speech-to-speech parallel pair dataset for language translation modelling are the MustC dataset (Cattoni, R., Di Gangi, M. A., Bentivogli, L., Negri, M., & Turchi, M., 2021), Open Parallel Corpus-Opus website, 50 Languages website (Limbu, 2020), Conversational Spanish-English dataset, Fisher Spanish-English (Jia et al., 2022, Jia, et al., 2019), LibriS2S (German-English) (Jeuris & Niehues, 2022), FLEURS with 1625 parallel Punjabi-English speech (Kaur et al., 2024). Another approach to generating parallel speech corpus is to utilize the ASR or TTS speech corpus with their transcripts where the speeches are synthetically generated (Cattoni et al., 2021; de Martos, 2022; Jia et al., 2022; Jia, et al., 2019). The majorly available sources of speech corpus for ASR and TTS tasks are the TED-LIUM2/3, LibriSpeech, Wall Street Journal (WSJ), Hub, CHiME4, CSJ, Aurora4, HKUST, and Fisher-CALLHOME Spanish. For this research work, speech corpus will be sourced from organisations that keep records of their voice signals. In this study, both the synthetically generated speech corpus of the pair and the naturally generated speech corpus of the pair will be considered.

## 2.5.2 Data Analysis

The collected parallel speech pair data is subjected to analysis to know the statistical properties of the data. Statistical properties that are considered will be the normal distribution of the speech corpus, the frequency of variations in the data, the mean, variances, etc. This ensures that it satisfies the quantity and quality requirements of the language-translation modelling, is able to train well, and obtains more insight into the characteristics of the dataset. This statistical analysis is carried out using SPSS software, MATLAB packages, or any statistical software package. Also, the quality of speech pair, background noise, and the number of hours of the audio recordings are analysed.

## 2.5.3 Data Preprocessing

This involves the conversion of raw speech of both the source and target to Mel-spectrogram features, after which discrete representations of both will be derived by using the HuBERT model or any other available Mel-spectrogram or raw speech to discrete conversion. This ensures that the discrete representations of both the source and target are available for training as input and target.

## 2.5.4 Data Cleaning

This involves the removal of unwanted audio samples from both the source and target speech files. Restructuring the speech corpus to be in the format needed for training, such as using the appropriate format and using the correct sampling frequency.

## 2.5.5 Feature Extraction Stage

This involves the extraction of speech feature vectors from the speech signals. The extracted feature vectors will be Mel-spectrogram representations of both the raw source input and target speech. Typical Mel-spectrograms are shown in Figures 2.8 through 2.10 of section 2.3.4.2.

## 2.5.6 Discretisation

Here, the extracted feature vectors will be converted to discrete representations for both the source and target extracted features to be used for training. This ensures reduced size for speech data.

## 2.5.7 Sequence-to-Sequence Modelling

This involves the training of the pre-processed parallel speech corpora of both the source and target language using the transformer architecture and other state of the art techniques highlighted in section 2.4. Thereafter, the developed trained model is tested for functionality and performance evaluation.

# 2.6 Performance Metric

## 2.6.1 Bilingual Evaluation Understudy

This is an objective evaluation that involves the comparison of the speech target output with that of the reference or ground truth. To compute the BLEU score, the output of the speech translation is fed to an ASR model to generate the text equivalent. These texts generated are then compared with the ground truth texts, and the BLEU score is computed. Mathematically, the BLEU score is computed using equation (2.24):

BLEU score= BP*expi=1Nwilogepi                                                                                 (2.24)

Where:

BP=Brevity Penalty=exp1- rc                                                                        (2.25)

Which is also computed as: BP=Brevity Penalty=min1,  rc

r=length of machine translated output (text or speech)

c=length of reference translation (text or speech)

pi=n-              gram modified precision score of order i, which is given in equation 2.26 as:

pi= Count Clip (matches, max-reference-count)candidatae n-gram                                                               (2.26)

N=maximum number of n-gram order to consider (usually up to 4)

wi=weight for n-gram precision of order

For instance, given the information below for both the machine-translated output (obtained using ASR) and reference output text:

Machine Translation (MT): The picture the picture by me.

Reference (Ref) 1: The picture is clicked by me. (used)

Reference (Ref) 2: This picture was clicked by me.

The analysis is indicated for each n-gram in Table 2.1.

Table 2.1: A summary Table for the Computation of BLEU score

| n = 1 |  |  |  | n = 2 |  |  |  | n = 3 |  |  |  | n = 4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MT l = 6 | Ref r = 6 | Min (MT, Ref) |  | MT | Ref | Min (MT, Ref) |  | MT | Ref | Min (MT, Ref) |  | MT | Ref | Min (MT, Ref) |
| “the” | 2 | 1 | 1 | “the picture” | 2 | 1 | 1 | “the picture the” | 1 | 0 | 0 | “the picture the picture” | 1 | 0 | 0 |
| “picture” | 2 | 1 | 1 | “picture the” | 1 | 0 | 0 | “picture the picture” | 1 | 0 | 0 | “picture the picture by” | 1 | 0 | 0 |
| “by” | 1 | 1 | 1 | “picture by” | 1 | 0 | 0 | “the picture by” | 1 | 0 | 0 | “the picture by me” | 1 | 0 | 0 |
| “me” | 1 | 1 | 1 | “by me” | 1 | 1 | 1 | “picture by me” | 1 | 0 | 0 |  |  |  |  |
| p1= 46= 23 |  |  |  | p2= 25 |  |  |  | p3= 04= 0 |  |  |  | p4= 03= 0 |  |  |  |

Hence, using equation (64), the Brevity Penalty is computed as:

BP=min1,  rc= min1,  66=min(1, 1)=1

Substituting into equation (63) with other parameters gives:

BLEU score= BP*expi=1Nwilogepi

BLEU score= 1*expi=14wilogepi

BLEU score= 1*exp(0.25loge23+ 0.25loge25+ 0 loge0+ 0loge0) [w1=w2=0.25, w3=0, w4=0];

BLEU score= 0.718=71.8

## 2.6.2 Mean Opinion Score

This is a subjective performance metric that is based on the judgment made by the observer on the output translated speech of the target language. It could be MOS Naturalness or MOS Similarity. In this type of subjective evaluation, raters are sought, who then score the output generated speech on a scale from 1 to 5 where 1 = Poor, 2 = Satisfactorily, 3 = Good, 4 = Very good, and 5 = Excellent. In MOS Naturalness, the raters judge the fluency and naturalness of the speech output, while in MOS Similarity, the raters score the output speech obtained by comparing it with a reference or ground truth output (which can be human-generated speech or synthesized one) on a scale of 1 to 5.

## 2.6.3 Statistical Evaluation

This refers to the utilisation of machine learning evaluation metrics. This involves Precision, Recall, F1-Score, etc.

I. Precision

This is the ratio of True Positive (TP) to the sum of True Positive (TP) and False Positive (FP). Mathematically, it is obtained as given in equation (2.27) as:

Precision= TPTP+FP                                                                                                                   (2.27)

II. Recall

This is the ratio of True Positive (TP) to the sum of True Positive (TP) and False Negative (FN). Mathematically, it is obtained as given in equation (2.28):

Recall= TPTP+FN                                                                                                                         (2.28)

III. F1-Score

This is the ratio of the product of precision and recall to the sum of precision and recall. Mathematically, it is obtained using equation (2.29) as:

F1-Score= 2*Precision*RecallPrecision+Recall                                                                                                (2.29)

IV. Computation of Precision, Recall, and F1-Score in Speech-Speech Translation

Given the reference text (Ground Truth) of the target language (Yorùbá) as RT and the Translated text equivalent of the translated speech of the target language (Yorùbá) of the output of the translator as MT as given below, the precision, recall, and F1-Score are calculated using equations (2.27), (2.28), and (2.29) as illustrated below (Hujon et al., 2022):

RT/GT: A bẹ̀rẹ̀ ìmúlò àjẹsára ibà pọ́njú-pọ́ntọ̀ ní ọdún 1938.

MT: A bẹ̀rẹ̀ ìmúlò àjẹsá ibà pọ́njú-pọ́ntọ̀ ọdúni 1938.

No of correctly translated words (bold)=6

Total number translated words in the output of the translator=8

Total number of reference words=9

Precision= No of correctly translated words (bold)Total number translated words =68 =0.75

Recall= No of correctly translated words (bold)Total number reference words =69 =0.66

F1-Score= 2*Precision*RecallPrecision+Recall= 2*0.75*0.660.75+0.66=0.702

# 2.7 Regularisation Techniques

The modelling of sequence-to-sequence translation involves the adjustment of training parameters to ensure the model generalises well for both the training and test cases (unseen data during training) and to prevent overfitting of the model. These are termed regularisation techniques. For deep learning, well-known regularisation techniques are L1 regularisation, L2 regularisation, normalisation, the addition of noise, denoising, volume normalisation, dropout, drop weight, data augmentation, etc. In both L1 and L2 regularisations, the goal is to adjust the weight within the model by setting the regularisation parameter high so as to reduce the weight coefficients.

# 2.8 Review of Related Works

## 2.8.1 Review of Literature on Dataset

In Gutkin et al., (2020), the authors presented a high-quality open-source speech corpus of the Yoruba language, which was released under a Creative Commons Attribution – ShareAlike 4.0 International (CC BY-SA 4.0) license and available for SOTA speech applications and academic, personal, and commercial use. According to the authors, the Yoruba language, with 11 other languages, can be traced to the Benue–Congo branch, a Yoruba sub-branch of the Niger-Congo family. The speech corpus contained a total of 4 hours, 1 minute, and 31 seconds of audio recordings sourced from 19 female and 17 male speakers. The average duration per recording was 3.9 secs and 4.2 secs for female and male speakers, respectively. The speakers within the age range of 21 to 43 years were sourced from Twitter feed and were asked to complete a voluntary form to participate in the process. The recording, which took place at two different centers (The office of LifeBank Nigeria in Yaba and The African Arts Foundation (AAF) in Victoria Island), was recorded using a Neuman KM184 diaphragm condenser cardioid microphone, which is a Blue ICICLE XLR to USB analog to digital (A/D) converter, with a sampling frequency of 48 KHz, and was saved as a 16-bit PCM RIFF wav file. The file has a total of 1,892 and 1,691 recordings (lines or sentences) for females and males, respectively; the minimum and maximum words (tokens) per sentence (recordings) were 2 and 23, respectively, for females, and 3 and 22, respectively, for males. The average words per sentence for males and females were 8.4 and 8.5, respectively. The total words for male and female were 14, 242, and 15, 880, respectively, while the total unique words were 3, 835, and 4, 113, respectively. Finally, to show the high-quality content of their speech corpus, it was demonstrated in a Simple Statistical Parametric Speech Synthesis (SPSS) scenario in which it was evaluated against the Yoruba Lagos –NWU Corpus (The first known available online Yoruba speech corpus) and related languages from CMU Wilderness dataset.

In Olatunji, T., Afonja, T., Yadavalli, A., Emezue, C. C., Singh, S., Dossou, B. F. P., Osuchukwu, J., Osei, S., Tonja, A. L., Etori, N., & Mbataku, C., (2023), the authors developed an ASR speech corpus (AfriSpeech) for speech research-related tasks. The focus of the dataset was on the accents of the speakers. According to them, there are no publicly available medically trained ASR systems taking African English accents into consideration and the nonexistence of accented speech data for African languages. Hence, their choice of developing an accented African speech corpus with transcripts for ASR model training and development. The data development started with sourcing texts from Wikitext-103 and web scraping. For the medical texts, they utilized PubMed and the NCBI disease datasets. The mobile application using Python/Flask was developed to collect the audio recording of the collected texts. Individuals were crowdsourced to record the speech equivalent of the text prompt on the web application with a payment of between $5 to $ 10. They could listen to their recordings to review before submission to the platform. Texts lesser than 5 characters or longer than 300 characters were removed as well as recordings lesser than 2 seconds or longer than 17 seconds. A total of 200 hours of accented generalized and medical English speech corpus with transcripts were developed that consisted of 13 countries, 67,577 clips, 120 Indigenous accents, and 2 463 speakers. The accents that did not consider North African were limited to Indo-European, Niger-Congo, Afro-Asiatic, Nilo-Saharan, and Khoe-Kwadi (Hainum) families. To evaluate the performance of their datasets, it was compared with the LibriSpeech test clean dataset. The AfriSpeech and LibriSpeech were evaluated on the HuBERT, Whisper, Wave2Vec, XLSR, conformer, and WavLM models, as well as ASR APIs such as GCP, AWS, and Microsoft Azure. They also finetuned Whisper medium and Wave2Vec large on general, medical, and combined domain datasets, and the WER of these models (6 pre-trained models, 3 APIs, and 2 pre-trained models) were compared. For instance, on Whisper large pre-trained model, LibriSpeech achieved WER of 0.167 on test clean while their finetuned Whisper medium and Wave2Vec large achieved WER of 0.351, 0.568, & 0.418 and 0.253, 0.415, & 0.314 for training with AfriSpeech-general, AfriSpeech-clinical, and AfriSpeech-combined respectively for the same LibriSpeech test clean. Utilising their AfriSpeech test dataset on the models, Whisper large had WER of 0.240, 0.375, & 0.306 on general, clinical, & combined datasets, respectively, fine-tuned Wave2Vec large (finetuned/trained on AfriSpeech-clinical) had WER of 0.424, 0.308, & 0.368 on general, clinical, and combined dataset respectively, while fine-tuned Whisper medium on combined dataset achieved WER of 0.192, 0.242, & 0.216 on general, clinical, and combined dataset respectively.

## 2.8.2 Review of Literature on Speech-To-Speech Translation

In Jia, et al., (2019), speech-to-speech translation using a neural network approach was carried out. The attention-based sequence-to-sequence neural network model was able to translate speech from one language to speech in another language. Translatotron, which offers the benefits of being able to retain voice, reducing errors propagated, handling untranslatable words, and possessing faster speed, consists of three components, which are the Optional speaker Encoder, sequence-to-sequence model, and Neural Vocoder. Adafactor optimiser with a batch size of 1024 was used for the training. Telephone conversational speech of Spanish Fisher corpus and its corresponding English translation, as well as another conversational corpus of read speech and parallel text pairs of Spanish-to-English translation (obtained through the reading of both sides of conversational Spanish-to-English MT dataset by crowdsourced humans), were utilised for the training. The performance of the Translatotron was carried out using BLUE score, translation quality, and subjective listening test using a 5-point Mean Opinion Score (MOS) for speaker similarity and naturalness for voice transfer. For the translation quality performance evaluation, a 16k Word-Piece attention-based ASR model trained on 960 hrs LibriSpeech dataset with WER of 13.4% and 4.7% on the test-other and test-clean sets, respectively, was utilised to obtain the text equivalent of the generated speech from Translatotron, and comparing the transcript generated with the ground truth reference translations. On the conversational Spanish-to-English dataset, 619 hrs target speech for the training was obtained using the English Tacotron 2 TTS model instead of the human target speech, in addition to the 1.4k hrs of source speech making whole 979k parallel utterance pairs. BLUE scores of 42.7, 48.7, and 74.7 were obtained for the Source+Target model, baseline model (STT & TTS cascade), and Ground truth, respectively.

In Radford et al., (2023),  the authors developed Whisper, which was a supervised model (weak) that was trained on about 680, 000 hrs. of multitasking and multilingual supervision of the dataset, which were explored and generalized well in relation to the standard benchmarks. An encoder-decoder architecture consisting of MLP was chosen for many speech-processing tasks, such as spoken language identification, speech translation, alignment, voice activity detection, and multilingual speech recognition. 39 to 1550 million parameters were utilized and trained using gradient norm clipping with a linear learning rate, which decayed to zero over the first 2048 updates after a warmup and AdamW Optimiser, and the model was trained for up to 220 updates with batch size of 256 segments. They developed different variants of Whisper such as Whisper tiny.en, Whisper tiny, Whisper base.en, Whisper base, Whisper small.en, Whisper small, Whisper medium.en, Whisper medium, Whisper large, and Whisper large-v2. Models were able to perform 74 language transcriptions and 98 language translations with English language transcription. The performance of the model was tested with other ASR models involving 4 commercials together with NVIDIA STT (CTC large), and it outperformed the best Open the NVIDIA model, an Open-sourced model. In comparison with unsupervised wav2vec 2.0 large (without LM) (Baevski, A., Zhou, H., Mohamed, A., & Auli, M., 2020), Whisper Large -v2 had WERs of 2.7 %, 4.0 %, 3.9 % on Supervised LibriSpeech, TEDLIUM, and WSJ respectively compared to 2.7 %, 10.5 %, and 7.7 % for wav2vec. For multilingual speech recognition, zero-shot Whisper outperformed XLS-R (IB) and mSLAM-CTC (2B) on the Multilingual LibriSpeech (MLS) dataset, where it had 7.3 % WER compared to 10.9 % & 9.7 % for XLS-R (IB) and mSLAM-CTC (2B) respectively.

In Chang et al., (2023), the authors looked into the topic titled Exploring speech recognition, translation, and understanding with discrete speech units. Unlike most research work that considers self-supervised speech features or mel-filter bank (MFCCs) as the feature extraction method for speech signals, they considered a new approach called discretisation where speech signals are converted to discrete representation equivalent of tokenised text, using different approaches. According to the authors, Vector Quantisation and K-Means clustering are some of these approaches. In this paper, they explored K-means clustering on the already extracted speech features where the cluster indices were used as the discrete representation of speech. However, it was observed that there was a temporal alignment between the discrete and original speech features. The experiments involve 12 ASR, 3 Speech Translations, and 1 Spoken Language Understanding. The seq-2-seq was used to map the discrete units to the target output. For the speech translation task, the MuST-C dataset (audio recording of transcription and translation from TED talks) was used. This dataset contains English-Dutch, English-Spanish, and English-French. The ESPnet toolkit was used for training. The setup was done for the encoder, which consists of a 12-layer EBranchformer, 256 hidden layers, with 4 heads in the self-attention. The decoder chosen was a 6-layer transformer decoder with 4 attention heads. The performance of the discrete representation of speech features was compared with that of the self-supervised learning (SSL) and mel-filter bank. It was observed that on the MuST-C En-De dataset, BLUE scores of 26.7, 28.6, and 29.7, respectively, were obtained for FBank, Discrete tokens, and SSL feature extraction approaches. For the MuST-C En-Fr dataset, BLUE scores of 37.1, 38.7, and 40.2 were obtained for FBank, Discrete tokens, and SSL feature extraction approaches, respectively.

In Limbu, (2020), Direct speech-to-speech translation exploring machine learning was carried out. The work carried out in this thesis was based on the Translatotron, a speech-to-speech model developed by Google (Jia, Weiss, et al., 2019). In this work, an encoder-decoder LSTM, which is a sequence-to-sequence (STS) network, was used to translate speech from Swedish to English and vice versa, unlike the conventional approach that involves text representations. Due to the unavailability of data, the author generated his own Swedish-English utterances using Google TTS API (which can be installed) on Swedish-English texts sourced from the Open Parallel Corpus-Opus website. Also, the Swedish-English speech corpus was sourced from the 50 Languages website, making two distinct datasets for the experiments. The building of the model, which involves data generation and cleaning, was carried out using Python 3.0 on Jupyter Lab Integrated Development Environment (IDE) and ran on the Google Colab Cloud platform for high computing processing and faster output results. Using the 50 Languages website dataset, for the first model, the model passed the first test of the first method, meaning it was able to memorise well but failed (resulted in white noise) the second test on the validation data, meaning it is not able to learn or generalise well on new data due to overfitting. Besides, the second method shows that the training loss is less than the validation loss, further depicting that the first model could not generalise well due to overfitting. For instance, at an epoch of 200, the training and validation loss were about 13.2 and 16.2, respectively. The training and validation loss for the second model were 11.9 and 14.7, respectively. With the introduction of the attention mechanism in the third model, the translation accuracy of target speech increases, but the difference between the loss vs epoch plots of the model and the previous immediate model was not that obvious.

In McArthur & Richardson, (2023), sequence-to-sequence speech translation from German to English language was carried out. It is an attention-based end-to-end model developed specifically for the aforementioned language pair, but it could be trained on any pair of languages. The Mel-spectrogram of German was fed as input to generate the target language Mel-spectrogram after which it is converted to the target speech waveform. The speech translation model developed was a modification to the architecture of Google's Translatotran model (Jia, Weiss et al., 2019) to make it easier and simpler to use. The LibriS2S speech-to-speech dataset that the human speaker generated to include the naturalness of the speech was utilised for training. The dataset consists of 40 hrs of German (De) recordings and corresponding 40 hrs of English (En) audio recordings making a total of 80 hrs of audio pair. Two models were developed; the baseline model, which utilizes the translatotron model with Tacotron code, was modelled and trained. Using TorchAudio for speech preprocessing, conversion from any file type of audio to Mel-spectrogram back to audio is easier to carry out. However, according to the author, the exploding gradient was experienced, which affected the training time of the baseline model. The second model was obtained via the altering of the base model's parameters and hyperparameters. Here the author first utilised TorchAudio for preprocessing of data for DataLoader, then adjustment made was the data normalisation in the DataLoader to make the data fall within 0 and 1, use of LeakyReLU activation function instead of tanh so as to solve the issue of exploding gradient, use of gradient clipping (value of 5) to avoid issues of the explosion, and finally, the regularisation techniques and learning rate were adjusted to ensure the model generalises well to unknown data. Then, the developed model was trained with a single sentence to ensure it also memorised that sentence in testing. The training on the full dataset was distributed on six Nvidia A100 GPUs. For performance evaluation, the output of the translator was fed to an ASR, and the generated texts were compared with the reference, and the BLUE score was computed. As initially stated, the baseline model using Translatotron and Tacotron yielded no results due to the exploding gradient. For single-sentence training, the predicted and target spectrogram plots showed similarity, and the predicted audio matched that of the reference (speech distinguishable to the human hear) after it had been converted to audio, even though some background noise could be heard. A BLUE score of 40.68 was obtained when an ASR was used to transcribe the predicted audio to text and then compared with the reference. The author noted that the BLUE score obtained was due to the used ASR and not due to the proposed second methodology implemented.

In Nyetanyane, J., & Masinde, M., (2019), with the aim of solving the language barriers experienced by non-Sesotho-speaking migrants from neighboring countries, the authors developed a speech translator from English to Sesotho language, which is an under-resourced language. According to the authors, most of the migrants in the Free State province of South Africa do not speak the language; hence, their choice is to develop UmobiTalk, which is a mobile-based speech translator for learning the Sesotho language English-speaking migrants. The parallel corpus utilised for the MT was centered on the major six language learning words, which are small talk, business, etiquette, greetings, tourism (places, culture, food, etc), and emergency words. For the usability evaluation, a sample of the population was selected, data was gathered about the platform, and this generalises to the entire population. The evaluation obtained from the selected sample of the population was compatibility, robustness, usability, and speed of the application. The researchers themselves conducted accuracy and speed experiments. The speed was based on the response time between the clicks of the application and when the output was obtained. According to them, the higher the number of texts the higher the response time and the smaller the speed and vice-versa. The accuracy was obtained by comparing the correct Sesotho texts with the machine-translated texts. An overall accuracy of 72.6 % and an overall translation speed of 57.5 % was obtained for UmobiTalk.

In Ayesha, M. L., Divya, M. O., & others, (2022), a single model that can translate several languages was developed with cutting-edge outcomes. The authors talked about the works Mark Zuckerberg, the co-founder of Meta, has implemented. It is in his interest to see how AI could be implemented for language translators. On February 23rd, 2022, Meta announced that it plans to develop an artificial intelligence (AI)-based translation system for anyone in the world. According to the authors, the top real-time translator applications are Google Translate, iTranslate app, Easy Language Translator, and iTranslate Voice 3. The Google Colab and cloud is a platform where Python could be easily implemented for various tasks. They implemented a sequence-to-sequence language translator that consists of multi-layered LSTM as the encoder to convert the input speech sequence into fixed-dimensional vector representation and fed to a decoder which consists of deep LSTM to decode the output sequence from the vector. Finally, the authors gave insight into the development of speech translators using the cascaded blocks of ASR, MT, and TTS using Python for implementation. According to them, the first step is to import required libraries such as speechrecognition, googletrans, pyttsx3 by first installing them and thereafter import them. For the first block, ASR functions for Define Recognition, Microphone Definition, and Recognise Speech were called. The result of the ASR is fed to the language translator block which consists of Define Translator functions that were called on the output of the ASR. Then the output translator is fed to the TTS Engines where functions are called for "Define Text to Speech Engine" and "Define Speaker Language," and the final step is to give the output of the MT as an argument to the TTS engine that has been configured. The benefit of this modelling program is its multilingual nature, i.e., being able to translate from one language to any language of one's choice.

In Kaur et al., (2024), the authors, in a quest to make contributions to speech translation research, developed a direct speech-to-speech translation model for one of the under-resourced Indic languages called Punjabi to English. The focus of the work was the utilisation of discrete representation of speech known as the discrete acoustic units as input to a transformer-based architecture. Hence, the developed model is called a Unit-to-Unit Translation (U2UT). The only publicly available parallel Punjabi-English dataset online is the FLEURS dataset, which consists of 1625 parallel speech corpus. Due to the insufficiency of the dataset, the authors obtained the Punjabi speech corpus and transcript using the Kathbath dataset and generated parallel English speech and text using the SeamlessM4T translation model. This shows that using this dataset, the Punjabi speech is naturally obtained while the corresponding English speech is synthetically generated, and both Punjabi speech and synthesised speech were sampled at 16 kHz. The discrete unit of speech for both the source and target utilised for the training were generated using the HuBERT Base model (6th-layered and 100 clusters), which was trained on 960 hours of Librispeech. According to the authors, the average number of discrete units in each sentence is about 300. Hence, they padded the discrete units with shorter sequences and trimmed discrete units with longer sequences so that a length of 300 was obtained. The pre-trained Vocoder called unit-based HiFi-GAN was utilized to convert discrete speech units to output audio waveform. For the model evaluation, the discrete unit of the test set with 3202 samples was generated and fed into the model to predict the target discrete units, after which it was converted to the target audio waveform by the Vocoder. To evaluate the performance of the U2UT model, the predicted English speech was fed to the Whisper large-v3 ASR model to generate texts that are compared with the ground truth text for the target language. Then, the BLUE score was evaluated. For comparison purposes, two other models were developed. First, the direct speech-to-unit translation (S2UT) consists of six encoder layers, six decoder layers, 4 heads attention, 2048 dimensions for the feed-forward, and 256 dimensions of the encoder embedding. The HuBERT Base model was utilised to generate the discrete unit of the target speech. The input to the system was the spectrogram of the source Punjabi speech, and the output of the decoder was the discrete unit representation of speech, which was fed to a Vocoder to generate the output audio waveform. Second was the cascaded system that consisted of ASR and MT that was obtained from Whisper large-v3 due to its multi-tasking capability. The Whisper ASR converted the speech input to generate Punjabi texts, which were later fed to the Whisper MT to generate the English text, which was compared with the ground truth texts to obtain its performance evaluation. It was obtained that the U2UT (BLEU score of 3.9829) model outperformed the S2UT model (BLEU score of 0.2954) by a BLEU score of 3.69. However, it was observed that both models performed lower compared to the cascaded system implemented with Whisper-large-v3 (BLEU score of 18.1136). This is because Whisper-large-v3 is a robust ASR that was trained on a large dataset of about 680,000 hrs of data. Hence, they suggested that if the dataset is increased beyond 83578 samples utilised for the present experiments, enhanced performance will be obtained. This U2UT model can be applied to any language pair other than Punjabi-English language translation.

In Rubenstein et al., (2023), the authors developed a large language model called AudioPaLM. The AudioPaLM is an integration of a text-based language model called PaLM-2 and a speech-based language model called AudioLM. The AudioPaLM was trained with both texts and audio files and could be used as a speech recognition system for transcribing speech-to-text, speech synthesis where speech is generated from texts, as well as speech-to-speech translation of various languages. The results show that AudioPaLM 8B AST, AudioPaLM 8B S2ST, AudioPaLM-2 8B AST, and AudioPaLM-2 8B cascaded ASR + translation had BLEU scores of 35.4, 36.2, 37.8, and 39.0 respectively on CoVoST2 dataset for AST task, whereas Whisper Large-v2 had 29.1 BLUE score. For zero-shot performance (using the FLEURS dataset), Whisper Large-v2 had a BLUE score of 23.3, while AudioPaLM-2 8B AST and AudioPaLM-2 8B AST had BLUE scores of 22.4 and 28.6, respectively. To check the quality of the generated speech, MOS was computed, where the subjective evaluation was calculated for both audio quality and voice similarity. Here, AudioPaLM was compared with Translatotron2 and CVSS-T, where it had a score of 3.65, while translatotron and CVSS-T had 3.36 and 3.41 scores, respectively, for objective evaluation on audio quality and values of 4.44, 3.96, and 3.88 for AudioPaLM, Translatotron2, and CVSS-T respectively on for subjective evaluation for audio quality. For voice similarity, AudioPaLM, Translatotron, and CVSS-T had subjective MOS scores of 4.00, 3.51, and 3.70, respectively. Thereafter, different experiments were performed, which are: training on multiple tasks, training from scratch vs. finetuning, different tokenisation schemes, training with combined tasks, training with additional speech-to-speech tasks, scaling the training data, decoding with AudioLM vs. SoundStorm, and impact of using architecture scale. For each of these experiments, the performance of the AudioPaLM was evaluated and it shows that varying the experiments tasks could generate different performance for the AudioPaLM. For instance, scaling the training data on AST and S2ST improves BLEU score performance but not WER performance improvement for ASR tasks.

In Jia et al., (2022), the author developed Translatotron2, an improvement on Tranlatotron1. It contains a speech encoder, a linguistic decoder, and an acoustic synthesiser, with a single attention mechanism that links them. The synthesiser consists of a duration predictor and upsampler, with RNN + Convolution. Just like Translatotron1, which generates speech and phonemes or texts as auxiliary outputs, Translatotron2 also has two outputs: The speech output from the synthesiser and the auxiliary output for phonemes of the target’s language from the decoder. It addresses the following problems of the original translation: noncontribution of the attention alignment learned by the auxiliary ST task on the S2ST task, challenges that occur due to two long spectrogram sequences with translation alignment modelling, and under-generation and over-generation issues known with attention-based speech generation. The dataset for the experiments was Conversational es-en, Fisher es-en, and multilingual CoVoST2 (es, fr, de, ca→en) datasets. Re-evaluating the Translatotron on Fisher and Conversational datasets and comparing it with Translatotron2, the translation quality shows that Translatotron2 outperformed Translatotron by 15.5 and 5.2 BLEU scores, respectively. Utilising the ConcatAug dataset lowered the BLUE scores from 16.4 to 0.4 and 8.4 to 3.7 for comparison between direct S2ST and cascaded system for both Fisher and Conversational datasets, respectively. Based on subjective MOS Naturalness for a single speaker's voice, it outperformed Translatotron, where it had 4.21±0.06, 4.19±0.06 with ConcatAug while Translatotron had 4.15±0.07 on the Conversational dataset. On Fisher dataset, it had scores of 3.98±0.08, 3.79±0.09 (with ConcatAug) while Translatotron had MOS of 3.70±0.08. For both cases of datasets, cascaded system & reference (synthetic) had values of 4.31 ± 0.06 & 3.37±0.09, and 4.04±0.08 & 3.95±0.07 on Conversational and Fisher datasets, respectively. On robustness, the robustness issues of over-generation in the predicted translation speech, like long pauses and babbling, were measured by an Unaligned Duration Ratio (UDR) with a 1-second threshold. Results showed that Translatotron2 has a UDR of 0.13 while Translatotron had 0.69 on the Conversational dataset, which showed it is about 7 times lower than Translatotron, which depicts it is more robust.

In Lee, A., Chen, P.-J., Wang, C., Gu, J., Popuri, S., Ma, X., Polyak, A., Adi, Y., He, Q., Tang, Y., Pino, J., & Hsu, W.-N., (2021), the authors worked on direct speech-to-speech translation with discrete units where a discrete speech encoder was applied to the target speech. It is a sequence-to-sequence, speech-to-unit translation (S2UT) where the source spectrogram is mapped to the target discrete representation during training and gets converted into speech by a converter (vocoder) using HiFi-GAN. The model was able to generate both the speech and texts (auxiliary task) as output. The auxiliary tasks are used during training. Using the Fisher Spanish-English dataset, the S2UT framework was able to achieve an improvement of 6.7 BLEU scores when compared to the baseline direct S2ST that predicts Mel-spectrogram features representations of the input Mel-spectrogram. To evaluate the translation quality, they obtained the text of the translated speech using an ASR system, and the BLEU scores were computed. For the MOS Naturalness, 200 utterances were collected, and 8 raters rated them on a scale of 1 to 5. For source and target written, on the development test, the cascaded baselines model of ASR+MT+TTS and S2T+TTS had BLEU scores of 42.1 and 38.5, respectively. The direct S2ST with transformer Translatotron had a BLEU score of 32.9, while S2UT had a BLEU score of 34.0. S2UT reduced with CTC and achieved a BLEU score of 38.2. When compared with Translatotron and Translatotron2, it outperformed them, where Translatotron with pre-trained encoder had a BLEU score of 30.1. For MOS Naturalness, the cascaded baselines model of ASR+MT+TTS and S2T+TTS had MOS of 3.37 plus or minus 0.15 and 3.43 plus or minus 0.14, respectively. The direct S2ST transformer Translatotron had a MOS score of 3.31 plus or minus 0.11, while S2UT, with no reduction, had a MOS score of 3.35±0.14. S2UT reduced with CTC achieved a MOS score of 3.41±0.14. When compared with Translatotron and Translatotron2, it outperformed them where Translatotron had MOS score of 3.69±0.07 while Translatotron2 had MOS of 3.98±0.08. For training with the source written and without target transcripts, on the development test set, cascaded baselines model of ASR+T2ST, ASR+T2UT had BLEU scores of 25.3 and 39.9 respectively, while Direct S2UT reduced had BLEU scores of 34.4, while Translatotron from literature had BLEU score of 7.4. For training without source transcripts and without target transcripts, Direct S2UT reduced achieved a BLEU score of 31.1. When compared with Translatotron, it outperformed Translatotron, which had a BLEU score of 0.4 on the development test set.

In de Martos, (2022), a cascaded S2ST was developed with a particular emphasis on the modelling of TTS for cross-lingual voice cloning of both seen and unseen speakers (zero-shot adaptation) of the generated output speech. In the work, the efficient incremental TTS model was designed and integrated with streaming ASR and simultaneous MT system to generate speech of the target language for Universitat Politècnica de València (UPV) media lecturers during live recordings of their lectures. 47 of the lecturers that generated the Dex TTS dataset participated in the evaluation, which involves Naturalness, Speaker Similarity, Real or Synthetic, and Survey. 50 samples were selected from the test set. MOS Naturalness of 4.1±0.1, 4.1±0.1, and 3.6±0.1 were obtained for synthesized Spanish, Catalan, and English speeches respectively MOS Similarity of 4.0±0.5, 4.0±0.2, and 3.4±0.2 were obtained for unseen cases of Spanish, Catalan, and English language respectively. The next experiment was carried out to address goal number 2 ,which deals with enhancing the robustness, controllability, and efficiency of modern SOTA TTS technology ,and this was achieved by using non-autoregressive attention-based TTS in place of the autoregressive attention-based model, in addition to this is the explicit modelling of energy and pitch (F0) in TTS model, as well as the use of GAN-based vocoder that has improved performance over WaveRNN, or WaveNET. For goal number 3, which involves simultaneous S2ST experiments, Europarl-ST v1.1 corpus , which is a multilingual spoken language translation dataset from the debates archive of the European Parliament (9 different official European languages),was utilised. Only English and Spanish languages were considered for both En-Es and Es-En translation. The focus of this experiment was on the incremental TTS that made cloning of the original speaker possible in the target language, i.e., retaining the speaker's voice in the English language when the source is Spanish language and vice versa. Streaming ASR achieved WER of 13.4 % and 10.7 % on English and Spanish speech corpora, respectively, and an offline WER of 12.7 % and 9.8 %, respectively. The simultaneous MT trained on the OPUS parallel public dataset achieved BLEU scores of 47.4 and 41.3 on English-Spanish and Spanish-English text-to-text MT,respectively. Its BLEU scores performance was compared with the fixed wait k policy for different values of k ,where it performed better than this policy. For incremental TTS, which was incorporated with speaker ID to generate speaker embeddings, language ID to generate language embeddings, phoneme sequence to generate phoneme embeddings, and speaker classifier, MOS Naturalness achieved BLEU scores of 4.15±0.20, and 4.17±0.19 on English and Spanish texts to waveforms respectively on full-sentence training. MOS Speaker Similarity achieved 3.84±0.22, and 3.67±0.21 on English and Spanish texts to speech, respectively on full-sentence training. The author achieved an acceptable latency range of 7 to 9 seconds, which is close to 3 to 6 seconds for human interpreters. The fourth goal, which involved zero-shot speaker adaptation experiments that involved inputting the model with unseen speakers during experiments, was achieved. The transfer learning approach for speaker encoders that had already been pre-trained on multiple speakers was integrated into the TTS models. An encoder/decoder block consisting of a stack of conformers was used for the TTS model of the cascaded system. The conformer block consists of a feed-forward module, a self-attention, a convolution module, and a second feed-forward module. A Mean Opinion Score (MOS) Naturalness of 2.8±0.06, 4.1±0.05 were obtained for conformer (baseline), conformer (Least Square GAN-LSGAN) models, respectively, and compared with ground truth values of 4.9±0.04, and 4.9±0.05 for Dex-TTS, and LibriTTS respectively.

In Niranjan et al., (2020), the authors developed whisper-to-natural speech-to-speech translation model using Whisper TIMIT (wTIMIT) Corpus and Speech Ocean (King-ASR-066 American English ASR Corpus). The architecture developed was an improved transformer architecture that consisted of three units: The first unit was the decoder block for the target phone prediction, the second unit was the encoder block for the source speech mel-spectrogram, and the last unit was the decoder block for the target speech mel-spectrogram. Each decoder unit was a stack of 6 decoder consisting of masked multi-head self attention, multi-head cross attention, and feed-forward network. The encoder consisted of multi-head self-attention, with feed-forward network and layer normalisation. Time stretching was performed on the source-target audio files to make them equal in length before being fed into the model for training. 18620 parallel training utterance of both the source and target speech of 15 hours was utilised for training; sampling of all utternace to sampling rate of 16 KKHz due to different sampling rate from the combined datasets. Both 80 dimensional MFCC features and 24 dimensional smooth spectral features generated using Librosa library were utilised for training. The BLUE scores and ASR WER were computed to evaluate the model.

In Gong et al., (2025), the authors developed speech-to-speech translation model from Tibetan to Chinese using encoder-decoder based sequence-to-sequence model (called here base transformer model). The training involves using the Tibetan audios source spectrograms as input to predict discrete Chinese units as output. The target hidden units were obtained using the HubERT model which was fine-tuned for the task and utilised for training. The model is a speech-to-unit translation (S2UT) model trained on the Tibetan mel-spectrogram features and Chinese discrete unit representation. The predicted Chinese discrete unit was fed to a vocoder to generate the Chinese audio waveforms.The model was able to capture linguistic differences which represents a direct end-to-end neural machine translation. To select the optimal set-up for the HuBERT model based on Phone-unit Normalized Mutual Information (PNMI) values, the perforrmance of HuBERT was evaluated on various configuration parameters such as number of layers, cluster size, etc. Via selection of the configuration of right parameters, HuBERT was fine-tuned and trained on the target Chinese mel-spectrogram for the training so as to generate the hidden discrete unit representation utilised for training.Auxiliary tasks such as Chinese texts, Tabitan character and Chinese character generations were integrated into the training to enhance the model performance. SOTA BLEU score and MOS Naturalness were computed to evaluate the model. Results showed promising translation quality from Tibetan to Chinese language. For instance, BLEU scores of 16.02, 19.34, 22.77, and 26.46 were obtained for base-transformer (original encoder-decoder model), base-transformer with HuBERT fine-tuning, base-transformer with auxiliary tasks and Base-transformer with HuBERT fine-tuning and auxiliary tasks respectively.

In Dong et al., (2022), the authors developed a speech-to-speech translation model using Spanish-English, and English-Chinese dataset. The model was an improved transformer architecture on the Google's original Traslatotron model. The authors utilised transformer encoder and decoder instead of the  RNN based encoder and decoder utilised in translatotron model. Two auxiliary tasks involving ASR to generate the target phone and automatic machine translation to generate the source phoneme were added just like in translatatron to aid the model performance. The mel-spectrograms of both the source and target speech were fed to the model during training, as well as the phonemes of both the source and target data. For the English ASR, they utilised Wave2vec and CTC models and Conformer model for the Chinese ASR. For data scarcity, they employed pseudo-labelling and results showed that the improved transformer based translatotron performed better than the original translatotron. ABLEU score of 42.4 was obtained for improved transformer-based translatotron using pseudo labeling in comparison to 24.8 obtained for translatotron model.

In Li et al., (2022), the authors developed a textless speech-to-speech translation model which is an improvement on translatotron 2. Auxiliary task was also added. Unlike in translatotron 2 where the target phonemes are being predicted by the auxiliary tasks, here, the auxiliary task is trained to predict the discrete speech reprrsentation. In comparison to translatotron 2, a VQ-VAE based speech quantizer was utilised to extract the discrete target speech obtained from the linguistic decoderso as to be guide the linguistic decoder during training. The extracted targetdiscrete units are compared with the discrete units obtained from the traget predicted mel-spectrogram. The use of a speech quantizer is to address the problem of when transcripts are not available for the target speech language to be utilised in training, hence, a self-supervised approach is needed to be used for training. A pre-trained w2v-BERT encoder was used unlike a model from scratch utilised in translatotron 2. On CVSS-C dataset, a BLEU score of 17.7 and 10.1 were obtained for textless translatotron 2 and translatotron 2 respectively.

In Shi et al., (2023), the focus of the speech-to-speech translation model was on the utilisation of different TTS models for generating different target speeches of the same traget language. The speech-to-speech model was a Speech-to-Unit that predicts the traget discrete unit. To carryout the experiments, the authors utlised three different TTS models-these are VITS, Fastspeech2, and Tacotron2. While tacotron2 and Fastspeech2 generate target spectrogram, VITS generate target waveform. They further experimented by using three set of vocoders as inputs to the Fastspeech2 and Tacotron2 that generate mel-spectrogram. The vocoders utilised were Hifi-GAN, WaveGAN, and StyleMelGAN. For training, the HuBERT model was utilised to transformed the target speech to target dsicret units, which is then fed with source mel-spectrogram. The training results was the predicted target discrete units.According to the authors, the proposed approach gave a 2.8 BLEU score improvements over the baseline Fisher Spanish-English dataset.

In Nachmani et al., (2023), the authors developed an unsupervised direct speech-to-speech translation model to translate from Spanish to English and vice-versa. The model is called Translatotron 3, which is an unsupervised approach when compared to the original Translatotron and Translatotron 2 developed by the same organisation (Google). Translatotron 3 solved one of the problems associated with supervised direct speech-to-speech translation dataset, which is the difficulty in sourcing parallel audios of both the source and target languages by utilising unpaired audios. The authors trained two separate models at the training phase: The source spectrogram as input to predict the source spectrogram as output via a source decoder and a target spectrogram as input to predict the target spectrogram as output via a target decoder. These two models utilised a shared encoder with each having its own decoder. The source decoder consisted of a linguistic decoder, synthesiser, and a single attention mechanism. The target decoder also has similar structures. After separately training both the source spectrogram and target spectrogram in an unsupervised fashion, they were merged into a single model during the second phase called the reconstruction phase, where the source spectrogram was utilised to predict the target spectrogram via a target decoder, and the target spectrogram was back-translated to predict the source spectrogram by inputting it into a source decoder. The final predicted source spectrogram was then compared with the original source spectrogram to minimise the loss. When low loss is obtained, it can be concluded that the translation from source spectrogram (English) to target spectrogram (Spanish) or back translation from target (Spanish) to source spectrogram (English) is correct. Three (3) major losses were computed for the model. These are the Multilingual Unsupervised Embedding (MUSE) loss, Reconstruction loss (Spectrogram Loss, Phoneme Loss, and Duration Loss), and a back-translation loss. The authors via ablation studies showed that removing some of these losses had greater effects on the BLEU score results. For instance, removing the reconstruction loss reduces the English to Spanish translation from 18.85 to 0.41; removing the back translation loss reduces the Spanish to English translation from 24.27 to 3.62. It was further shown that Translatotron 3 performed better than a cascaded model with 18.14 BLEU score on synthesised CVSS dataset. It also achieved +0.75 BLEU score improvement over cascaded approach on real or natural CoVoST2 test set. However, its performance is closer to the supervised approached but considering the difficulty of the architecture, the results are worth nothing.

In Julius et al., (2024), English-to-Yoruba Speech-to-Speech translation was carried out using improved transformer architecture with designed augmented attention mechanism to enhance translation accuracy. The key improvement among the components of the transformer architecture as earlier mentioned was the attention mechanism utilized. In contrast to a single value (obtained from a single weight value matrix) used to multiply the attention weight / score in the original transformer architecture, the authors projected the weight matrix of the value into two sub weight matrix values (for instance splitting along the y dimension) and multiply the original value with each of the two sub weight matrices to obtain two new values, V1, and V2. what followed was the multiplication of the two sub-values, then scaling with the square root of the original value size, then multiplication of softmax of this result with the original value to obtain a new value similar in dimension to the original value and this was later utilized to multiply the attention weight-this to ensure more representations of the input signals are captured during training considering the structure and complexity of Yoruba language with different diacritics, intonation, prosody, etc., in the training audio signals. Using 36 layers in the transformer for both the encoder and decoder, with 42 attention heads, the developed model was trained on about 200, 023 audios, obtained from the native speakers as well as the public database to achieve a BLEU score, MOS, and translation of accuracy of 0.7, 2.8, and 98 % respectively. The existing transformer model architecture, has translation accuracy of 88.16 %.

## 2.8.3 Review of Literature on Utilisation of Machine Translation in the Medical World

In Hudelson & Chappuis, (2024), the authors stated that MT applications are being used informally in health facilities where human interpreters are not available. Due to the accuracy and accessibility of this tool, healthcare professionals need guidance on how and when to safely use the tools, as well as mitigate the potential risks that come with it. The objective of the work was to explore the potential factors that could facilitate or hinder communications when using voice-to-voice MT. According to them, the underuse of medical professional interpreters is due to factors such as availability, cost, underestimation of patients' language skills, and difficulties in scheduling. The use of MT comes with errors generated, which pose challenges in the health sector as sensitivity is highly taken seriously. However, a move from the use of statistical MT to neural MT has significantly improved the quality of results obtained for MT. The participants in this experiment are the Geneva University Hospitals, Switzerland's staff, where 14 health professionals that, consists of 6 nurses, 5 doctors, and 3 allied health professionals working in the Primary Care Division, conducted 60 consultations (56 with Microsoft Translate and 4 with Pocketalk W) with their patients using one of the two voice-to-voice MT apps in 18 languages, out of which 15 consultations (13 with Microsoft Translate and 2 with Pocketalk W) were observed. The observed languages were English, Portuguese, Arabic, Spanish, Romanian, Ukrainian, Russian, and Turkish. The participants were instructed to either install the Microsoft Translate app on their smartphone devices or use Pocketalk W (PW) as an alternative for those who were unable or not willing to install the Microsoft app on their devices. The questionnaire utilised comprises 8 closed post-MT questions plus comments for the medical professionals, 4 questions for the patients asked by the professionals using the app, and 7 questions for the patients asked by the observer using the app. The observers were also health professionals who observed errors in communication while using the app, the comfortability of both parties while engaging the app, whether there was eye contact, etc. Results of the experiments showed that the professionals were satisfied with MT communications in only 53.8 %, with 82.7 % of consultation goals achieved. All 4 consultations attempted with Pocketalk W in Tamil, English, Albania, and Italian were completely unsatisfactory due to technical difficulties, such as delays in the audio translation due to cellular network or Wi-Fi of PW, poor results obtained in the speech recognition stage of the translator leading to poor or nonsensical translation. Consequent upon this, they decided to present the result of the experiment with the Microsoft Translate app on 52 consultations for 13 languages.

In Tu et al., (2024), a conversational text-based diagnostic AI called Articulate Medical Intelligence Explorer (AMIE), a Large Language Model (LLM) that could increase quality of care, accessibility, and consistency was developed by the researchers-this involves more work to be carried out on such system as the desire to make the AI diagnostic dialogue tool to approximate clinician expertise is an outstanding challenge. AMIE was developed with the help of training data sourced from medical reasoning multi-choice questions (MedQA), which consists of 11,450 questions for training, 1,273 questions for the test, and 191 curated questions; summaries of electronic health record (EHR); expert-curated long-form medical question answering, and real-world conversations (which resulted in 89,027 conversations for training and 9,892 for validation). The framework consisted of a Vignette Generator, a Simulated Dialogue Generator (a doctor agent, patient agent, and moderator), and a Self-play Critic. A chain of reasoning, which comprises analysing patient information, formulating a response and action, and refining the response, was incorporated into the AI-based diagnostic tool. The AMIE diagnostic-based tool can perform better in terms of diagnostic accuracy, management reasoning, history-taking, empathy, and communication skills. Utilizing the style of an Objective Structured Clinical Examination (OSCE), Experiments involve simulated patient actors interacting with AMIE and simulated patient actors interacting with 20 Primary Care Physicians (PCPs) using text-based consultations with validated patient actors in a randomised crossover study. The 149 case scenarios used for the study were obtained from clinical facilities from UK, Canada, and India, and patient actors and specialist physicians carried out its evaluation. Results showed that AMIE achieved superior performance and greater diagnostic accuracies over human PCP on 24 out of 26 and 28 out of 32 cases considered according to the patient actors and specialist physicians (AMIE & 23 selected specialist physicians), respectively, while it is non-inferior on the rest of the cases. The limitation of the work is the use of unfamiliar synchronous text-chat, which the clinicians were not used to, that allows LLM patients' interaction but is not a representation of the usual clinical practice.

## 2.8.4 Summary of Literature Review on Speech-To-Speech Synthesis

Table 2.2: Summary of Literature Review on Speech-To-Speech Synthesis

| REFERENCE | METHODS | DATASETS/LANGUAGE | RESULTS | FEATURE EXTRACTION METHODS | ADVANTAGES / LIMITATIONS |
| --- | --- | --- | --- | --- | --- |
| (Chang et al., 2023) | Sequence-to-sequence modelling using Convolution Sub-Sampling | Dataset: MuST-C dataset: Average of 430 hours of audio recordings of TED talks. Languages: English-German English-Spanish English-French | BLUE scores using FBaank: En-De: 26.7 En-Es: 31.2 En-Fr: 37.1 BLUE scores using Discrete tokens: En-De: 28.6 En-Es: 33.0 En-Fr: 38.7 | (1) Use of discretisation for speech extracted features. (2) De-duplication and sub-word modelling were used to further reduce the size of the speech features. | Advantages 1. Performance enhancement due to reduction in the size of the speech features (implementation of discrete tokens). 2. Reduction in training time due to discrete tokens. Limitations 1. The work did not consider the comparison of other discretisation approaches, such as Vector Quantisation, with the K-Means clustering utilised for discretisation. |
| (Limbu, 2020) | Sequence-to-Sequence modelling with a multi-head attention mechanism | Datasets: 1. Open Parallel Corpus-Opus website 2. 50 Languages website Language: Swedish-English | 50 Languages website dataset Model 1(LSTMs encoder & decoders) Training loss: 13.2 Validation loss: 16.2 Model 2 (BiLSTMs encoder & decoder): Training loss: 11.9 Validation loss: 14.7 | Mel-spectrogram (MFCCs) | Advantages 1. It explores different model training. 2. It involves the use of multi-head attention for enhanced performance. Limitation 1. It did not take the speaker's features such as prosody, voice, accents, etc. Into consideration. 2. The computational time of the algorithm was not considered. |

Table 2.3: Summary of Literature Review on Speech-To-Speech Synthesis (Cont.)

| REFERENCE | METHODS | DATASETS/LANGUAGE | RESULTS | FEATURE EXTRACTION METHODS | ADVANTAGES / LIMITATIONS |
| --- | --- | --- | --- | --- | --- |
| (Jia, Weiss, et al., 2019) | Attention Sequence-to-Sequence approach with: Speaker Encoder and Neural Vocoder Translatotron1 | Datasets: Conversational Spanish-to-English dataset Fisher Spanish-to-English dataset: Language Spanish-English | Conversational Spanish-to-English dataset: BLUE scores: Source + Target model: 42.7, Cascaded model (STT & TTS): 48.7, Ground truth: 74.7 Fisher Spanish-to-English BLUE scores: Source + Target model with 1 attention head: 24.2, Source + Target model with 4 attention heads: 31.5, | Mel-spectrogram (MFCCs) | Advantages 1. It explores different model trainings. 2. It utilises multi-head attention for speech-to-speech translation. 3. it utilises two (2) datasets for comparison Limitation 1. The computational time of the algorithm was not considered. 2. Challenges encountered with cross-language voice transfer. 3. Performance was lesser than the cascaded system. |
| (Jia et al., 2022) | Direct speech-to-speech Translatotron 2 | Datasets: Conversational es→en, Fisher es→en, Multilingual CoVoST2 (es, fr, de, ca→en) Languages: Spanish-English French-English German-English Catalan-English | Translation Quality Fisher dataset: Translatotron2 outperformed Translatotron by 15.5 BLEU score. Subjective MOS Naturalness for single speaker’s voice: Fisher dataset: Translatotron: 3.70±0.08 Translatotron2: 3.98±0.08 | Mel-spectrogram features | Problems it addresses from the original Translatotron: 1. Non-contribution of the attention alignment learned by the auxiliary ST task on the S2ST task. 2. Challenges that occur due to two long spectrogram sequences with translation alignment modelling. Advantages 1. Performance was closer to the cascaded system, where the translation quality reduced from 16 to 0.4 BLEU score. Limitations 1. it does not consider the discrete representation of speech. |

Table 2.4: Summary of Literature Review on Speech-To-Speech Synthesis (Cont.)

| REFERENCE | METHODS | DATASETS/LANGUAGE | RESULTS | FEATURE EXTRACTION METHODS | ADVANTAGES / LIMITATIONS |
| --- | --- | --- | --- | --- | --- |
| (McArthur & Richardson, 2023) | Direct Speech to speech-to-speech translation, | Datasets: The LibriS2S speech-to-speech dataset Language: German-English | Model 1: Translatotron + Tacotron No results were obtained due to exploding gradients Model 2: Single Sentence Training BLUE score: 40.68 for a single sentence. | Mel-spectrogram Utilises TorchAudio for speech preprocessing of input spectrogram. | Advantage 1. The work did put prosody, voice, and emotion into consideration. 2. Data normalisation technique was implemented. Limitation 1. Unreliability of the ASRBlue score. 2. The work did not consider the use of other performance measures, such as BLAZER for text-free |
| (Nyetanyane & Masinde, 2019) | ASR+NLU+MT+TTS UmobiTalk Application | Datasets: Sesotho Speakers Language: English-Sesotho | Overall Translation Accuracy: 72.6% Overall Translation Speed: 57.5% | Feature Extraction was mentioned for the ASR, but the type was not stated. | Advantages: 1. It serves to solve the language barriers experienced by non-Sesotho migrant speakers. Limitations: 1. It is a one-way translation from English to Sesotho language and not the reverse. |
| (Ayesha et al., 2022) | ASR+MT+TTS Python Code for implementation The End-to-end approach was also discussed | Datasets: Not mentioned Language: Multi-lingual (from one language to any language.) | Not mentioned Just the Python code for implementation was discussed. Installed libraries: pip install speechrecognition pyttsx3 googletrans | Not mentioned | Advantages of end-to-end approach: 1. it reduces latency and avoids mismatch. Advantages of the Python codes for the cascaded system: 1. it is multi-lingual Disadvantages 1. The paper did not discuss the technologies behind the speech-to-speech translation |

Table 2.5: Summary of Literature Review on Speech-To-Speech Synthesis (Cont.)

| REFERENCE | METHODS | DATASETS/LANGUAGE | RESULTS | FEATURE EXTRACTION METHODS | ADVANTAGES / LIMITATIONS |
| --- | --- | --- | --- | --- | --- |
| (Kaur et al., 2024) | Sequence-to-sequence model: Direct Unit-to-Unit Translation (U2UT) model | Datasets: 1. FLEURS with 1625 parallel Punjabi-English speech. 2. Synthetically generated English speech using SeamlessM4T translator from the Kathbath dataset. Language: Punjabi-English | U2UT Model: BLUE score: 3.9829 S2UT Model: BLUE score: 0.2954 Cascaded Model (Whisper-large-v3): BLUE score: 18.1136 | U2UT Model: No feature extraction was utilised. Rather, a discrete representation of raw audio speech was obtained by passing raw audio speech into the HuBERT Base model trained on 960 hours of Librispeech | Advantages: 1. The use of discrete speech units that reduce the dimension of the speech signal while also preserving the original speech contents. Limitations: 1. The dataset size was insufficient for training; hence, the performance of the proposed approach was lower than the cascaded system implemented with Whisper-large-v3. 2. There was a limitation of available computing. |
| (Rubenstein et. al., 2023) | Discretisation + Decoder-only transformer model Models: AudioPaLM 8B AST, AudioPaLM 8B S2ST, AudioPaLM-2 8B AST, AudioPaLM-2 8B cascaded ASR + translation | Datasets: CoVoST2, CVSS, VoxPopuli, Common Voice, Conversational ExEn, YouTube ASR, WMT/TED TTS, PaLM MT TTS Language: Nonspecific | CoVost2 Dataset (AST Task): BLEU scores: AudioPaLM 8B AST: 35.4, AudioPaLM 8B S2ST: 36.2, AudioPaLM-2 8B AST: 37.8, AudioPaLM-2 8B cascaded ASR + translation: 39.0 Whisper Large-v2: 29.1 | No feature extraction Discrete tokenisation | Advantages: 1. It performed better than the SOTA Whisper-Large-v2 Model. 2. It utilises discrete tokens for training. Limitations: 1. It is not a sequence-to-sequence modelling, 2. There might be possible delays introduced during training. |

Table 2.6: Summary of Literature Review on Speech-To-Speech Synthesis (Cont.)

| REFERENCE | METHODS | DATASETS/LANGUAGE | RESULTS | FEATURE EXTRACTION METHODS | ADVANTAGES / LIMITATIONS |
| --- | --- | --- | --- | --- | --- |
| (Lee et al., 2021) | Sequence-to-sequence modelling: Speech-to-Unit Translation (S2UT) | Datasets: Fisher Spanish-English dataset Language Spanish-English | Training with source transcripts, target transcripts, source speech, target speech Dev Testset (BLEU score): ASR+MT+TTS: 42.1 S2T+TTS: 38.5 Direct S2ST with transformer Translatotron: 32.9 Direct S2UT: 34.0 S2UT reduced with CTC: 38.2 Translatotron with pre-trained encoder: 30.1 transcripts, source speech, target speech Direct S2UT reduced: 31.1. Translatotron: 0.4 | Mel-Filterbank with 10ms frame size utilised for developing models (cascaded systems of ASR+MT+TTS, STT+TTS, S2ST) compared with S2UT. Discrete representation of the target speech carried out on the target English speech | Advantages: 1. Reduced size of the direct S2ST due to the discrete representation of speech, hence the name S2UT. 2. The work explores training with source speech, source transcripts, target speech, and target transcripts and other situations when either transcript was present. Limitations: 1. Paralinguistic features present in speech, such as prosody, voice, and emotion, were not taken into consideration during training. 2. The other models developed are complex compared to other existing direct S2ST model |
| (de Martos, 2022) | Cascaded S2ST: Streaming ASR+Simultaneous MT+ Incremental TTS Tacotron 2 TTS remodified to allow cross-lingual voice cloning WaveRNN | Datasets: Dex TTS (Consists of 98 UPV Lecturers in English, Catalan, and Spanish): To train the Modified Tacotron 2 TTS model Europarl dataset OPUS Parallel Public and | ASR (WER): Streaming and online: English speech corpus: 13.4 % Spanish speech corpus: 10.7 % Offline: English speech corpus: 12.7 % Spanish speech corpus: 9.8 % MT (BLEU SCORES): English-Spanish text-to-text: 47.4 Spanish-English text-to-text: 41.3 | Mel-spectrogram: 100-bin log magnitude, hanning window length: 50 ms, Frame size: 12.5 ms, FFT: 1024-point | Advantages: 1. Robust, controlled, and efficient text-to-speech system. 2. It enables the voice transfer of the original speaker to the target language by retaining the emotion, voice, and prosody (rhythm, intonation) of the speech. Limitations Delays encountered in the cascaded system were designed with reduced value in comparison to human interpreters. |
| (Julius et al., 2024) | End-to-end transformer architecture with improved or augmented attention | Datasets: Publicly available dataset & dataset curated using native speakers’ recordings Language: English-Yoruba | Average BLEU score: 70 Translation accuracy: 98 % MOS: 2.8 | Mel-spectrogram-based model | Advantages: 1. The model incorporated an enhanced attention mechanism to capture more input representation. 2. The model performed better than the original transformer model. Limitation: 1. The complexity of the model which took about 73 hours for training in comparison to 71 hours for the existing transformer model. 2. The number of audios is limited to 200,023. |

# 2.9 Gaps in existing knowledge

Based on the literature reviewed so far for the speech-to-speech modelling, which also involves dataset sourcing, the existing gaps identified in the literature are summarised in Tables 2.7 to 2.8

Table 2.7: Summary of Gaps in Literature

| Consideration | Authors | Gaps |
| --- | --- | --- |
| Dataset | (Kaur et al., 2024); (Cherednichenko, 2020); (Grabar & Cardon, 2019); (Qian et al., 2023); (Kaur et al., 2024) | Insufficient amount of medical and nonmedical parallel speech corpus for the modelling of speech-to-speech translation Difficulty in acquiring parallel speech corpus |
| Speech-to-speech Development | (Meyer et al., 2022); | Lack of development of speech-to-speech translation for Indigenous African language |
| Speech discretisation | (Chang et al., 2023) (Rubenstein, et. al, 2023). | Not much work had been done Possible generation of errors due to lack of discretisation |

Table 2.8: Summary of Gaps in Literatures (Cont.)

| Consideration | Authors | Gaps |
| --- | --- | --- |
| Speech features | (Limbu, 2020) (A. Lee et al., 2021) | Speech paralinguistic features such as the prosody (intonation), accent, and emotion are not mostly taken into consideration |
| Cross-lingual voice transfer | (Jia, Weiss, et al., 2019); (Kaur et al., 2024) | Challenges are encountered with cross-language voice transfer, and performance was lesser than the cascaded system |
| Complexity of the architectures | (Jia et al., 2022) | The system is complex due to the architecture implemented, and the computational complexity of the models is not mostly stated |
| Training data contents | (Nyetanyane & Masinde, 2019) | The inclusion of words for the MT parallel corpus was limited to six major languages, which are etiquette, small talk, greetings, business, tourism (places, culture, food, etc), and emergency words |
| Computing | (Kaur et al., 2024) | Limitation of available computing |

In this work, the unavailability of dataset, speech discretization, cross lingual voice transfer, and computing complexities are major gaps found in existing literature to be filled. The curation of dataset exercises will facilitate availability of English-Yoruba speech corporal for speech-to-speech translations. The discretization which will be carried out using the HuBERT model will ensure reduced complexity of the model and sourcing individuals to do the English and corresponding Yoruba recordings will ensure the speakers’ voices and accents are transfer from English to Yoruba.

# CHAPTER THREE

# METHODOLOGY

# 3.1 Introduction

The development of speech-to-speech synthesis for English-to-Yorùbá language involves several stages, as highlighted in chapter 2. Different approaches could be utilised to achieve different results; however, for SOTA results, the utilization of the deep learning approach has made this possible. This chapter highlights the tools to be utilised to achieve this work. Coupled with this is the proposed architecture that gives an in-depth understanding of the model to be developed, the roles of various parts of this work, and the methodology to be implemented to achieve the aim.

# 3.2 Summary of Methodology

Table 3.1: Summary of proposed methodology

| Specific Objectives | Methodology | Data | Expected Results |
| --- | --- | --- | --- |
| 1 | Sourcing of transcripts; Utilisation of text-to-text translator; recording of audio files using a web site; Use of SPSS/MATLAB for analysis. | Text transcript, Text-to-text translator, Human being | Quality English-Yorùbá parallel corpus |
| 2 | Sampling, Feature extraction: MFCCs Discretisation: Hubert Modelling, K-means clustering – fine-tuning and training of HubERT model. | Outputs from Objective 1 | Discretized features with reduced dimensions that are ready to be fed as input to the model. |
| 3 | Improved Sequence-to-modelling using transformer architecture and training with torchaudio and nn tools of Pytorch. | Outputs from Objective 2 | State-of-the-art English-Yorùbá language translation model with speaker voice being preserved. |
| 4 | Utilisation of various software packages like Python, Flask, HTML, CSS, APIs, etc. | Output from Objective 3 | User-friendly Softbots language translator. |
| 5 | Computation of BLEU scores, ROUGE-L, MOS Naturalness, and MOS Similarity, as well as testing of Softbot by selected individual | Outputs from Objectives 3 and 4 | SOTA BLEU scores, ROUGE-L, and MOS that are typical of S2ST models. |

# 3.3 The Proposed Direct Speech-to-Speech Architecture

The architecture to be implemented in this proposed S2ST is an extension to Translatotron 2, a speech-to-speech translation model that was trained on conversational English to Spanish, Fisher English to Spanish datasets as well as Multilingual CoVoST2 (es, fr, de, ca→en) datsetes. The Translatotron 2 is shown in Figure 2.7. The proposed architecture is shown in Figure 3.1.

Figure 3.1: The Proposed Speech-To-Speech Architecture (An extension to Translatotron 2)

Figure 3.2: The Speech Synthesizer of Figure 3.1

## 3.3.1 General Overview of the Proposed Architecture

The proposed architecture, which is an extension to Translatotron2, is shown in Figure 3.1 and its synthesizer in Figure 3.2. As illustrated in Figure 3.1, the input to the architecture is an English waveform whose Yoruba Equivalent is to be synthesised. The waveforms are passed through an MFCC block, where the speech feature vectors will be extracted. The extracted speech features are fed to the discretisation block, where the reduction in the size of the speech feature is carried out while the features are preserved. Present-day State-of-the-art speech-to-speech modelling explores discretisation approaches (Chang et al., 2023; Rubenstein et al., 2023). The discrete representation of the target speech (Yoruba) will also be obtained, and both will be fed into the model during training. The discretised speech feature vectors will be fed to an encoder, stacks of LSTM, BiLSTM, MLP, conformer, etc. The encoder processes these input vectors, and the encoded vector output is fed to a single multi-head attention mechanism. The attention mechanism ensures a proper alignment between the English spectrogram and Yoruba texts (Jia et al., 2022). A single attention mechanism will be utilised, unlike in the original Translatotron, where three different attention mechanisms were used. One was for the S2ST task, and the other was for ST tasks for both English and Spanish phonemes. The attention output is fed to two blocks, i.e. the linguistic decoder and acoustic decoder blocks. The linguistic decoder block outputs the Yoruba phonemes while the acoustic decoder block, which is called a synthesizer according to Figure 3.1., converts the outputs of both the linguistic decoder and attention blocks into predicted Yoruba speech units. During training, the predicted Yoruba speech units will be compared with target Yoruba units to obtain the prediction loss. A detailed representation of the synthesizer is shown in Figure 3.2. It consists of an upsampler, duration predictor, and an RNN block (LSTM, BiLSTM) with a convolutional block. To obtain the Yoruba waveform, the speech units obtained are fed to a neural vocoder (WaveRNN, Graffin Lim Algorithm, etc.) that does the conversion to Yoruba audio, which is a replica of the inputted English audio waveform. Each of the blocks is explained thus:

## 3.3.2 Source and Target Speeches

One of the extensions to Translatotron 2 architecture, which this work addresses, is the utilisation of a different parallel speech corporal consisting of English and Yoruba as the source and target datasets, respectively. Unlike Translatotron 2, which is multilingual and was trained with Spanish, French, German, Catalan, and English language datasets, this proposed architecture is bilingual. The accents of both the English and Yoruba corporal will be taken into consideration during the collection of audio samples from crowd-sourced individuals.

## 3.3.3 Mel-frequency Cepstral Coefficients

This refers to the frequency analysis of the speech signals that are used to extract relevant features of the speech signal. The output of MFCC is 39 features, where the first 13 represent the MFCC features (the first 12 features represent the transformed spectrogram features and the 13th feature represent the energy of the speech for that frame), and the second and last 13 features each represent the first and second derivatives, respectively. The pipeline for MFCC feature extraction is shown in Figure 3.3.

Figure 3.3: MFCC Feature Extraction Process

As shown in Figure 3.3, the input signal waveform (English) will be digitised by sampling at 16 kHz to obtain the digital equivalent. The audio features will be sliced into frames, and audio samples will be extracted within the window size set. Typical frame and window sizes are 10 ms and 25 ms, 10 ms and 20 ms, as illustrated in Figure 3.4

Figure 3.4: Frame and Window Size Illustration

The sliced frame is modelled using equation (3.1)

xn=wns[n]                                                                                                                             (3.1)

Where:

xn=sliced frame

wn=window size which can be modelled using hamming, hanning window etc

sn=original audio signal

The window is modelled using equation (3.2)

wn=1-α-αcos2πnL-1                                                                                                  (3.2)

Where:

α=0.46164 for Hamming and α=0.5 for Hanning

L=window width

The windowed frame is fed to a DFT, which will be modelled using equation (2.9)

For faster computation and reduction in the complexity of DFT, a FFT will be implemented. The FFT reduced the complexity of the DFT by reducing redundant coefficients in the DFT through matrix computation.  The next step is to compute the logarithm of the DFT output. This represents the spectral features obtained from the input speech waveforms. This is represented in equation (2.7)

The next step in the block to model the speech features to be utilized for this proposed work is the computation of the Mel filter bank, whose outputs are Mel-filter bank features. This is modelled using equation (3.3)

YrRm= k=1L-1Wmk|XrRk|2                                                                                               (3.3)

Where:

|XrRk|2=power of the speech at each frequency=DFT Power Spectrum

Here, triangular band-pass filters are applied to convert the frequency information so as to mimic what human ears can perceive. The output of equation (3.3) is a Mel-scale power spectrum. Finally, the DCT, which produces uncorrelated features, is taken to obtain the first 12 feature vectors representing the speech waveform. The DCT is modelled using equation (3.4)

YrRn= m=0M-1log(YrR[m])cos⁡(n(m+0.5)πM)                                                                     (3.4)

Where:

YrRm=mel filter ank output

The utilisation of uncorrelated features makes the ML models easier to model and train.

The 13th feature represents the energy in each frame and is modelled using equation (3.5)

Energy= t=t1t2x2[t]                                                                                                              (3.5)

The second 13 features, which measure the changes in features from the previous frame to the next frame, are modelled using equation (3.6).

d1t= ct+1-c(t-1)2                                                                                                                 (3.6)

Where:

d1t=delta value of the MFCC features=13 features

ct-1=features from the previous frame

ct+1=features from the next frame

And the last 13 MFCC features are modelled using equation (3.7)

d2t= ct+2-c(t+1)2                                                                                                               (3.7)

Where:

d2t=delta value of the MFCC features=13 features

ct+1=features from the previous frame

ct+2=features from the next frame

## 3.3.4 Discretisation

The goal of the S2ST task is to obtain an SOTA target output. One of the present approaches to obtaining this is to utilise speech discretisation. The discrete representation of speech lowers the computational cost and memory usage. Here HuBERT model (Hsu et. al., 2021) will be utilised on the Mel-spectrogram features to generate the discrete representation of both the source and target language. The HuBERT model which has been pretrained on 960 hours of LibriSpeech corpus will be utilised. The 6th layer of the HuBERT model with a certain number of clusters (e.g., 100) will be set to generate the discrete units of both the English and Yoruba languages. Each cluster point represents the discrete representation of speech. The discrete representation of speech was not used in Translatotron 2 architecture. Other approaches to performing discretisation are w2v-BERT, Universal Speech Models USM-v1, and USM-v2.

## 3.3.5 Encoder

In Translatotron 2, the encoder utilised was a stack of conformer blocks that consisted of a feed-forward layer, self-attention layer, convolutional layer, and second feed-forward layer. In this proposal, the implementation will be done using a stack of LSTMs or BiLSTMs, as well as an improved transformer encoder. The transformer encoder in the original paper (Vaswani et al., 2017) uses a single multi-head self-attention that captures how much relevance each input discrete speech has for each other. In this work, multi-head self-attention will also be considered.

## 3.3.6 Attention Mechanism

This is a cross attention that finds the alignment between the encoder output and the multi-head self-attention output of the decoder (in this case linguistic decoder). A single multi-head attention that captures both the linguistic and acoustic information in speech and ensures acoustic and linguistic alignment between the source English and target Yoruba language will be used in this proposed work. Unlike in the original Translatotron architecture, training with an attention mechanism for S2ST tasks ensures the model learns well during training. Another benefit of using an attention mechanism is that it ensures the synthesizer sees both the linguistic and acoustic features for the discrete output and then ensures the paralinguistic and non-linguistic features of speech are preserved at the output Yoruba waveform (Jia et al., 2019; Jia et al., 2022). The attention mechanism is modelled using equations (2.21) and (2.22).

## 3.3.7 Speech Synthesizer

This decoder is fed with two inputs-phoneme decoder and attention outputs. As shown in Figure 3.2, it consists of an upsampler, duration predictor, and the main decoder (RNN + Convolutional block as utilised) in Translatotron 2. In this proposed work, LSTM, or BiLSTM with Convolutional block, will be implemented, as well as an improved transformer decoder with a masked multi-head self-attention, cross attention and feed forward layer with layer norms. This speech synthesizer is an acoustic synthesizer that generates the predicted Yoruba speech units. It works by predicting the duration of each element of the input sequence and subsequently upsampling them. The upsampled speech unit is fed to the main decoder, which will be an LSTM or BiLSTM, to obtain the target speech unit of Yoruba and then to the convolutional layer for further refinement of the output speech units.

## 3.3.8 Decoder

This works like a decoder in ST or ASR tasks. It takes the representation of the linguistic speech units from the cross attention mechanism and produces a phoneme representation replica of the source English speech units. The design of this linguistic decoder will be carried out using a stack of LSTMs or BiLSTMs, which will be utilised for the last stage (decoder) of the speech synthesizer. The linguistic decoder will also the carried out using the transformer decoder. The purpose of the neural vocoder is to generate the expected Yoruba waveform. This can be implemented using the well-known WaveRNN, HiFi-GAN or Graffin Lim algorithm to convert the Yoruba speech units to Yoruba waveforms.

# 3.4 Neural Machine Translation Method

As summarized in Table 3.1 and detailed in the block diagram of Figure 3.5 and flow chart of Figure 3.10, the stages for the development of the English-to-Yorùbá language translation resource involve the data collection, analysis, and data pre-processing stage, training of an improved transformer architecture as well as transfer learning model, development of a prototype SoftBot, and performance evaluation of the overall system (speech-to-speech translation model, SoftBot interface for user engagement).

Figure 3.5: Methodology Flow Approach

## 3.4.1 Data Curation

The English- Yorùba speech-to-speech translation model as previously stated requires sufficient amount of training parallel corporal. As there is presently no available English- Yorùba parallel corporal in the public domain, there is the need to curate dataset for training. Similar work carried out in (Julius et al., 2024) had their English-Yoruba parallel speech corpora curated themselves. This section details the stages embarked upon for the curation of data from the highlights of the resources needed to the development of a user friendly portal and sourcing of participants to record their voices for this task.

### 3.4.1.1 Tools

A) General and public health database

The exploration and searching of relevant databases were embarked upon to search materials to extract texts or transcripts. This involved accessing storage records that have these texts that were  made available to the participating individual for recording. For this work, public health and general domain databases were searched. For general domain, this involved Wikipedia, available texts transcripts datasets such as Leipzip (Yorùba texts), Mozilla Common Voice (English texts), Developing an open source (Yorùba texts), MuST-C (English texts), LibriSpeech (English texts) etc. Databases such as National Center for Biotechnology Information (NCBI) (English texts), PubMed, a section of the NCBI for biomedical literature, World Health Organisation (WHO), Word Bank, British Broadcasting Cooperation (BBC), Biomedical Malaria, Malaria Consortium, Malaria Report Nigeria, Malaria Symptoms File, Ted Talk, Youtube, and popular Nigeria newspapers such as Daily Times, Daily Independence, Vanguard, Guardian etc. were searched through for public health malaria related English text transcripts (Olatunji et al., 2023).

B) Text-to-text translator

This refers the use of State-of-the-Art (SOTA) text-to-text translator such as the Google translate, Microsoft translator, DeepL translator, Baidu translate, Amazon translate, open source text-to-text translators such as Moses, KantanMT, OpenNMT, Apertium, and other well-known translators. For this task, the Google translator was used to translate from English to Yorùba and vice versa.

C) Web development tools:

The user-interface for the speech-to-speech translation model must be user friendly. This can be achieved using web development tools such as Hyper Text Mark-up Language (HTML), Cascaded Style Sheet (CSS), JavaScript, React. Coupled to this is the testing and deployment stage using Flask application, and well known server such as Vercel as used in this work.

- Web Site Development

This involves the utilization of HTML, CSS and JavaScript. They are web development tools for developing the user friendly interface. The HTML 5 version with all its tags are used to design the interface of the website. The CSS 3 is used to add aesthetics to the HTML pages. The JavaScript is used to add some functionality to the HTML Pages. For this work, the HTML 5, CSS 3, and JavaScript were the web development tools used to develop the a four-page user friendly website called ASPMIR SpeechTech Crowdsourcing Portal.

- Flask application

This is a Python framework that is used to simulate a real time server deployment. It is a package with its functionality tailored to the HTML pages. It works with requirements and json files with the HTML pages and a Python file. In this work it was used to initially simulate the real time user engagement where the functionality of the pages were tested.

- Vercel deployment

This is a real time online server for the deployment of the developed flask application. It functions as the backend via which user information can be accessed when input. Details about the login, user statistics, such as user visit per day can be accessed via this application. It was used for the deployment of the flask applications containing 4 HTML pages, 2 JavaScript files, requirements file (dependencies), and a vercel.json file.

D) Crowd-sourced Participants

This involves the human participants to record their voices for the selected texts. Each participant will record the texts available in English and Yorùba equivalent for a certain number of transcripts allocated to each. For this work, individual were projected with a recording of 200 voices in English and Yoruba equivalent.

### 3.4.1.2 Steps to Curation of Data

A) Searching of online databases for public health and general domain texts

As highlighted above, online newspapers such as Daily Times, Leadership news, Vanguagrd etc, World Bank, WHO, NCBI, PubMed in the public health domain, as well as ASR database such as LibriSpeech, Mozilla Common voice, MuST-C etc were first sourced to find relevant malaria related transcripts and general texts respectively. The diversity in sourcing different databases is to ensure the transcripts are rich in different contents that represent real world scenario and project a generalization of the machine learning speech-to-speech translation model to be developed. For the general domain transcripts, Yorùba texts were obtained from Leipzip and Developing an open source databases. English transcripts were gotten from the Mozilla Common Voice, MuST-C, and LibriSpeech. In the case of the public health texts, English texts were obtained from all the aforementioned databases.

B) Pre-Processing of Obtained Transcripts

The transcripts obtained from ASR databases such as Leizip, Iroyin Speech, Mozilla Common Voice, Must-C, and LibriSpeech were already in sentence form arranged line by line with other information such as the duration of the recorded speech for each texts. Python codes were developed to extract on the texts from each file in their respective databases and saved with.txt extension. All the transcripts in the public health domain including the WHO, World Bank, NCBI, PubMed, online newspaper outlet were in raw form in the form of long sentences and paragraphs with punctuation marks and unneeded characters; as a publication paper etc. They needed to be transformed into sentences of an average of within 14 to 120 characters (Cattoni et al., 2021). Different Python codes were written tailored to the format of the raw nature of the respective database files extracted, and were used to filter out sentences lesser than or greater than the ranges earlier mentioned. The Python codes were carefully developed to ensure the content of the raw sentences were not lost. Breaking of the original texts was done at the period, “.”, some at comma, “,”, and some at “and” to generate different sentences from the same text file. They were eventually saved as texted files.

C) Development of both source and target texts

English texts generated from the raw transcripts were translated to their Yorùba equivalent using Google translate, and vice versa. Some of the words in the obtained Google translate were re-written using hands due to poor translation given to them by the translator. Examples of such include naming words that were translated such as names of places or things like Ade to crown whereas Ade as used in the sentence refers to a name of a person, words containing mountain in Yoruba but their English translations do not permit them to be altered for example the translation of “Oladejo Okediji, gbajúgbajà òǹkọ̀wé Yorùbá papòdà ní ọdún tó kọjá”. To ensure the texts were rich in Nigeria names, places, things, some of nouns in the sentences were changed to Nigeria names of places, things, human etc via the Python Codes developed. Examples of such names utilized were change of other country names such as Algeria, Malaysia, Cabo Verde, Mexico, South Africa, Ecaudor, etc, to Nigeria, replacement of foreign names to Nigerian names such as Àdùnní, Ọlọ́runyọmí, Kúnlé, Ìwàlọlá, Ẹniafẹ́, Ìfẹ́lódùn, Ọládiípọ̀, Tàlàbí, Akíntọ́lá, etc, replacement of foreign names of places to Nigeria names of places like Àbújá, Ikọ̀tún, Èbúté Mẹ́ta, Ìjẹ̀bú, Ìsẹ́yìn, Òǹdó, Sáàgámù, etc, and inclusion of names of things like gaàrí, mọ́in-mọ́in, gààrí, Jàlabíà, Bùgan etc (Olatunji et al., 2023). The Yorùba number system sourced (Adeniyi et al., n.d.; Elesemoyo, 2025; Overmann, 2025) were also included to ensure, the developed texts were rich in indigenous Yorùba number system, and were also translated to English texts manually since Google translator was not given the correct Yoruba number system translations as expected.

D) Development of web page for audio recording

The next step in the curation of the parallel speech corpora is the development of a user friendly web site called ASPMIR SpeechTech Crowdsourcing Portal. The web site consists of 4 web pages including the Landing, Yorùba Corpus page, English Corpus, and Transcripts page.

The Landing Page

The Landing page contains information about the portal, and highlights detailed information to the intended crowd-sourced participants. A user feedback form was also included on this page to get user experience per day during the recording exercises.

[Figure 3.6: The Home Page]

The English Corpus Page

The English Corpus page is where the users are expected to record their voices for the English audios. It has a recording bottom that the users engage with. This consists of Start, Pause and Stop bottom. Once activated by a user, the microphone on the smart devices which can a laptop, a mobile phone is activated to start the recording. A user may decide to pause the recording probably due to interruption, and resume at a later time by pressing the Pause bottom twice to resume from where they stop, and finally a stop bottom to end the recording for that particular transcripts. The recorded audio is saved as a .wav extension with a sampling rate of 48 KHz. Information on how to make selection of texts from the storage via the transcripts page, saving of recorded audio etc., are also given in this page. The page also contains some text areas for user engagement for pasting their intended texts to be recorded. A link to the audio storage space is also provided on this page. It also contains recording audio information such as length, type (general or public health), accent, and file named.

[Figure 3.7: The English Corpus Page]

The Yoruba Corpus Page

This is the page where users are expected to record their Yorùba audio of the selected texts. It works similar to the English Corpus page and same interface. Users may decide to do the recording of the English and corresponding Yorùba audios on the Yorùba Corpus page or both on the English Corpus page based on their choices.

[Figure 3.8: The Yoruba Corpus Page]

Transcripts Page

Via the transcripts page, the participants have access to the storage database where texts transcripts of the English and Yorùba equivalent are stored. The transcripts page contains information on how to navigate the page to access these files. It has hyper links to the general domain texts storage, public health domain texts storage, English Corpus page, Yorùba Corpus page, and Home page, as well as links to some selected databases such as NCBI, Mozilla Common Voice, PubMed, Youtube Videos etc.

[Figure 3.9: The Transcript Page]

D) Development of Manifest Files

Manifest files are files that contains the recording and texts information of each audio files. In this work, 60 % of the files audio files are transcripts are allocated to the public health domain data since this is the focus of the work. The remaining 40 % is the for general domain audio recordings. A manifest file could be prepared using a normal text editor, json file, yaml file etc. The manifest files were developed for each speaker since each speaker is projected to record 200 audios of English and corresponding 200 Yorùba audios. A typical example of the manifest file developed is using text editor is:

{

"metadata": {

"project_name": "General Speech Corpus",

"discreption": "A speech corpus for general related terms and discussions",

"version": "1.0",

"speaker_id": "SPK_060",

"domain": "General",

"language": "English",

"sampling_rate": "16000"

},

"audio_files": [

{

"file_name": "LIBRISPEECH_PARTA_1_40_ENG.wav",

"file_path":  "INSTITUTION_1_UNILAG/GENERAL/SPK_060/ENGLISH/AUDIOS/LIBRISPEECH_PARTA_1_40_ENG.wav",

"transcription": "If you can give me just a minute, Richard, before you go out",

"audio_duration": 15 secs

},

{

"file_name": "LIBRISPEECH_PARTA_1_41_ENG.wav",

"file_path":   "INSTITUTION_1_UNILAG/GENERAL/SPK_060/ENGLISH/AUDIOS/LIBRISPEECH_PARTA_1_41_ENG.wav",

"transcription": "She hesitated a little over the hard worked word",

"audio_duration": 12 secs

},

]

}

E) Focus Language

The source and target languages whose speech corpora were curated are English and Yoruba, respectively. For the state-of-the-art result to be obtained, the accents of the speakers were taken into consideration. The Oyo Yorùba dialect was taken into consideration which is to be spoken across different Yorùba accents. The Oyo Yorùba is chosen as is the official Yorùba language used mostly in official functions.

F) Sourcing of participants

Participants were sourced from 10 institutions within the Southwestern region of Nigeria and two institutions outside Nigeria. The 12 institutions are Covenant University, Ota, Ogun State, Nigeria; University of Lagos, Lagos State, Nigeria; Lagos State University, Lagos State, Nigeria; Federal University of Agriculture Abeokuta (FUNAAB), Ogun State, Nigeria; University of Ibadan, Oyo State, Nigeria; Obafemi Awolowo University, Ife, Osun State, Nigeria; University of Ado-Ekiti, Ekiti State, Nigeria; University of Ilorin, Kwara State, Nigeria; Federal University of Technology, (FUTA), Akure, Ondo State, Nigeria; University of Abomey-Calavi, Benin Republic; and University of Lome (UL), Togo. The choice of these locations was based on dominancy of the Yorùba ethnic group within these locations. Participants within the age range of 18 years and above including staff and students were engaged per institutions within a 5-working days and each participant is tasked with 200 audio recordings of English and corresponding Yorùba audio recordings. Each participant is rewarded with a minimum of $ 5 (Olatunji et al., 2023).

G) Audio Recording

The curation of speech data involves recording selected text files made available to the crowdsourced individuals. The recording were carried out for this task using the developed online web application (Gutkin et al., 2020b; Olatunji et al., 2023) to accelerate the curation exercises. Participants were directed to the storage location where the text files are stored to select the lines they were expected to record (for both English and the corresponding Yoruba texts). Participants were expected to identify or label each audio file with the same tag on the text file with the addition of “_ENG” for English and “_YOR” for Yoruba. For instance, if English and corresponding Yoruba texts read: YORMOZPARTA_1_2, the audio files read: YORMOZPARTA_1_2_ENG, and YORMOZPARTA_1_2_YOR for English and Yorùba audio files respectively (Cattoni et al., 2021). This ensures easy identification and orderliness and aids in the training of the ML models developed. Each participant were tasked to produce 200 audio recordings of both English and Yoruba texts. They were duly notified to take the accent (Olatunji et al., 2023) into consideration during each recording because the project lays more emphasis on the accent of the speakers as well as the speech.

H) Annotation Instruction

Recorders were tasked to read out punctuation marks in full such as saying “full stop”, “comma”, “question mark,” etc., where appropriate, while their accent is maintained, ensure that what is read out is the same as what is on the transcripts, reading out terms such as “centimeter per meter” instead of “centimeter slash meter”, “CXR” should be pronounced as “Chest Xray” and not “see ex arr”, “mmHg” should be pronounced as “millimeters of mercury, etc. never to read fast taking a break at a certain fixed time every day and recording upto a certain length of audio speeches per day, and ability to use their tone while recording (Olatunji et al., 2023). Recorders were also given a text-to-speech model that they could run to get correct pronunciation of some words (particularly the malaria related words) to be sure of using the exact pronunciation in the recording as illustrated below:

import pyttsx3

synthesizer = pyttsx3.init()

synthesizer.say("It also happens to be responsible for more than 95% of malaria cases in Nigeria.")

synthesizer.runAndWait()

synthesizer.stop()

I) Storage of Curated Corpora

Each recorder's curated audio recordings per day were stored on dedicated Virtual Machine (VM) within the FEDGEN platform (https://fedgen.net). It was duly monitored to ensure each crowd-sourced participant select the correct texts allocated to them and that the English and Yorùba audios are at the same level (same content in both languages).

### 3.4.1.3 Other Steps for Data Curation

This details other engagements embarked upon for the successful curation of the English-Yorùba speech parallel corpora. It involves the application for the ethical approval, and development of an informed consent form

A) Application for Ethical Approval

The ethical approval was obtained from the Covenant Health Research Ethics Committee (CHREC). The CHREC is duly registered with the National Health Reseraach Ethics Committee of Nigeria (NHREC). The requirements for ethical approval includes an application form, an abridged CV of the Supervisor, inform consent form, a proposal, and online training called (Collaborative Initiative Training Institutes, CITI), and a payment of #5000.00. All these requirements were met and an ethical approval (certification & letter of approval) was granted prior to commencement of the recording exercises.

B) Development of informed consent form

This is the 2-page form that details the information about the curation exercises on one page, and a consent form on the second page. The template obtained from the CHREC was used to develop the participants’ informed consent form.

C) Letter of Introduction

As the name implies, it contains information that introduces what the exercises are all about. It gives the crowd-sourced individual to know about the researcher such as the institution, supervisor, area of interest, affiliation, Etc. it consists of the following: introduction, project overview, purpose of data collection, target population, data collection methods, confidentiality, and anonymity, as well as contact information of the researcher.

## 3.4.2 Analysis of Curated Data

Here, well-known SPSS/MATLAB will be used to know the total duration of the parallel audio files, the mean, median, and variance, and check for the normality of the dataset. Also, each parallel corpus will be tested to ensure they carry the same narrative and will be properly labelled. The texts transcripts of those curated speech will also be analysed to extract information such as the variance, mean, mode, total number of characters, total number of words or tokens per text.  These are then fed to the pre-processing stage of the pipeline.

## 3.4.3 Pre-Processing of the Analysed Data

This will involve the use of a Python library for speech processing, such as TorchAudio, which contains a Mel-spectrogram, inverse spectrogram transformation, Graffin Lim algorithm model, etc. The TorchAudio is a module inside the Pytorch library. Other pre-processing to be carried out are normalisation, removal of silence, discretisation, etc. These functions can also be carried out using the librosa library.

### 3.4.3.1 HuBERT Discretisation

Both the English and Yoruba will first be pre-processed to extract the Mel-spectrograms as mentioned above. These dataset will be fed to a HuBERT pre-trained model and fine-tuned during training for the intended task to obtained discrete representations of both English and Yoruba Mel-spectrograms.

## 3.4.4 Improved Transformer Model Development

The speech-to-speech translation model, Translatotron 2 developed in the anchor paper (Jia et al., 2022) uses a sequence-to-sequence model using encoder with attention mechanism, a linguistic decoder and a speech synthesizer (speech decoder). In this proposed work, a SOTA transformer architecture will be developed. In the original transformer model (Vaswani et al., 2017), the author fed positional embedding vectors to a multi-head attention whose output is connected to a Layer norm. The layer norm is fed to a feed forward network and then to a Layer norm. This forms an encoder block and many encoder can be stacked to train a speech model. At the decoder side, the target positional embedding vectors are fed to masked multi-head attention, then to a Layer norm. The output of this layer norm and encoder are fed to a Cross Attention (just an attention network) where the key and value are from the transformer encoder, and the query is from the transformer decoder. Many decoder can also be concatenated. The output of the attention is fed to a Layer norm and then to a Feed forward network and finally to a layer norm.

In this work the nn tool module of the torch library of Pytorch will be used to develop improved transformer architectures tailored to the speech-to-speech translation model. The speech discretisation will be utilised on the extracted Mel spectrogram English and Yoruba speech features and the discretised speech features will be utilised for training to ensure enhance model performance.

## 3.4.5 Training

For training the model, the speech corpora of English and Yoruba datasets will be split into training and test set. About 70 % of the datasets will be for training and the remaining 30 % for testing.

### 3.4.5.1 Model Training Parameters

The hyper-parameters to be set for the model training are highlighted in Tables 3.2.

Table 3.2: Model Hyperparameters for Training

| S/N | HYPERPARAMETERS | VALUES |
| --- | --- | --- |
| 1 | Type of deep learning architectures for the encoder | LSTMs or BiLSTM or Conformer or MLP |
| 2 | Type of deep learning architectures for the decoder | LSTMs or BiLSTM or Conformer or MLP |
| 3 | No of LSTMs/BiLSTM/MLP for the encoder | Varying (e.g. 6) |
| 4 | No of LSTMs/BiLSTM/MLP for the decoder | Varying(e.g. 4) |
| 5 | Input spectrogram: Frame size window size Mel channels Sampling rate Mel lower bound Mel upper bound | e.g. 10 ms e.g. 30 ms e.g. 80 16 kHz 125 Hz 3800 Hz |
| 6 | Output spectrogram: Frame size window size Mel channels Sampling rate Mel lower bound Mel upper bound | e.g. 12 ms e.g. 50 ms e.g. 128 e.g. 24 kHz e.g. 20 Hz e.g. 12000 Hz |
| 7 | Encoder: BiLSTM Dimension Attention head | Varying e.g. 4 |
| 8 | Cross attention: Output dimension Hidden dimension Attention heads (multi-head) Dropout rate | Varying Varying 6 0.1 |
| 9 | Decoder: LSTM dimension Attention head | Varying e.g. 4 |
| 10 | Learning rate | e.g. 0.1 |
| 11 | Dropout rate | e.g. 0.24 |
| 12 | Optimiser | Adam optimiser, AdaDelta etc |
| 13 | Regularisation | L1 or L2 |
| 14 | Normalisation |  |
| 15 | Activation functions for the encoder, hidden layers, decoder | e.g. Softmax, ReLU, Softmax |
| 16 | Loss function | Cross entropy |
| 17 | Convolutional layer |  |
| 18 | Discretisation | e.g. HuBERT model with k = 50, layer = 6 |
| 19 | Epoch | Varying, based on training |

### 3.4.5.2 Base Model Training and Transfer Learning of Non-Medical Speech Corpus

A) Base Model Training with Single Training Instance

This will involve training from scratch by setting all the parameters and hyper-parameters, as illustrated in Table 3.1. For this experiment, a model will be developed for the single audio file to see how the output speech will be perceived and to leverage the high computing ability of the platform (FEDGEN) to be utilised due to the single data involved. Here, the input and output spectrograms will be plotted and compared, thereafter evaluated to see how they vary from each other. Here, the hyper-parameters will be varied to see how the output speech will be at each instance of variation of these parameters (McArthur & Richardson, 2023).

B) Base Model Training with Full Dataset

For the full dataset training, the whole dataset will be utilised in stages. The first phase will use about 50 % for training, the next will use about 70 %, and the last phase will use the whole dataset for training. The performance of each of the developed models will be evaluated to see how the output of Yoruba speech is changing (McArthur & Richardson, 2023).

C) Transfer Learning Approach to modelling Non-Medical Speech Corpus

Due to the low-resourced nature of the Yoruba language, and the inability to acquire parallel English-Yoruba speech pair, the transfer learning approach will be utilised. A well-known architecture like the Whisper Framework, AudioPalm, PolyVoice, SeamlessM4T, etc., will be utilised, where the knowledge learned for the developing of the speech-to-speech for any of these models will be transferred to training the little available English-Yoruba language pair corpus. The parameters of the transfer learning model will also be set prior to learning. Caution will also be taken to ensure the sampling frequency of the raw audio corresponds to that of the framework to be utilized, as well as setting other parameters on the architecture to be the same as that used for the development of the model architecture itself.

### 3.4.5.3 Base Model Training and Transfer Learning of Medical Speech Corpus

This will also involve single-sentence language and full training, as was earlier carried out for non-medical speech corpus. Here, the processed medical speech corpus will be fed as input to the model, after which training will commence. Base model training with single training instant and full dataset, and transfer learning model will take a similar direction to section 3.4.5.2, and hyper-parameters will also be varied for different ablation studies.

## 3.4.6 Testing

The testing datasets will be fed to the developed trained models under different experimental studies and the output Yoruba speech will be compared with the ground truth to ensure the model generalises well to unknown data.

## 3.4.7 Evaluation

### 3.4.7.1 Performance Comparison of Base and Transfer Learning Models for Non-Medical Speech

The performance of the different experimental setups for the single and full training of the non-medical speech corpus will be evaluated using the spectrogram plot, BLEU scores, ROUGE-L, and MOS computed. Comparison will be made between the different experimental setups for single training instances, between different experimental setups for full training, and between the best-performed single and full training experimental setup for the base model and the transfer learning models. The evaluation will also be carried out between the best-performed single training of the base model, full training of the base model, and transfer learning models. To know if the models will generalize well when fed with test data, it will see in training, the loss function of the training, and test/validation will be evaluated.

### 3.4.7.2 Performance Comparison of Base and Transfer Learning Models for Medical Speech Parallel Pair Corpus

Evaluations will be carried out using spectrograms of both the source and target language, MOS, ROUGE-L and BLEU scores, as highlighted in section 3.4.7.1. Performance will also be evaluated between the best-selected models obtained from the non-medical dataset training and medical dataset training. The MOS Naturalness and Similarity will be evaluated, as well as the BLEU scores and ROUGE-L.

## 3.4.8 Flow Chart of Neural Machine Translation Approach

The flowchart of the neural machine translation approach to modelling the proposed speech-speech language translation is displayed in Figure 3.10.

Figure 3.10: Flow Chart for the proposed Speech-to-Speech Neural Machine Translation Model

### 3.4.8.1 Flow Chart Explained

As shown in Figure 3.10, the flow chart of the proposed English-to-Yoruba starts with the collection of the parallel corporal of English and Yoruba meant for training the proposed transformer architecture. The collected parallel corporal is preprocessed using Python torch audio module of the Pytorch library. The MFCC features representation of speech are extracted and what comes next is the utilization of discretization to convert the vector representation of speech features to discrete representation. The discretized speech features of both languages are split into training, validation and evaluation datasets. The developed model is fed to the training dataset where it is used to develop models. The transfer learning model is also utilized at this stage. The results of the model training is checked if the output discretized Yoruba speech or the maximum epoch set is obtained. A decision is set at this point to either proceed to the validation of the model or retraining of the model. If yes, the model is fed with the validation datasets, and if no, the model is retrained. After validating the model, the model is then fed with the test datasets for evaluation. A decision block is also utilized at this point whether to proceed to the model deployment or retraining of the model. If no, the hyper-parameters of the model are adjusted and model is retrained, and if yes, the model is deployed on the prototype SoftBot developed for user engagement.

# 3.5 Prototype Development

## 3.5.1 Software Packages

To develop the prototype SoftBot, a friendly user interface for obtaining Yoruba speech of the inputted English speech, HTML 5, CSS 3, JavaScript with Flask application or Django and an online application deployment will be utilized.

## 3.5.2 Prototype Softbot

The prototype Softbot, which will be utilised by the user, will offer two real-time services. One is the voice recognition (ASR) service, and the other is the translation (S2ST) service specifically for engagement between medical practitioners and patients as well as public health practitioners and the public. The interface, which will be user-friendly, will ensure these two services are well taken care of. It will be designed for both online and offline capabilities.

## 3.5.3 Steps to Prototype SoftBot Development

### 3.5.3.1 SoftBot’s Functionality

A) Purpose and Goal of the SoftBot

The prototype SoftBot developed is to ensure a real-time experience of speech-to-speech translation from English to Yoruba language. It is intended to simulate a real-time experience of translation use case from one language (English) to another target language (Yoruba).

B) Target Audience and Needs

The prototype SoftBot is to be utilized by the stakeholders in the academic environments which includes students, lecturers, teachers, etc., general public for educational purposes and public health practitioners for their day-to-day engagement for educational (including malaria) translation use case.

C) Features of the SoftBot

Input: The input to the SoftBot is the English audio files

Output: The SoftBot is developed to offer tow (2) services: Speech-to-speech translation from English to Yoruba and SR services. Hence the outputs of the interface are Yoruba audios and Yoruba texts

D) User Registration and Feedback: The SoftBot also contains a user registration page to monitor how much people utilize the page per day as well as a user feedback form for getting user response meant for improving the interface and services provided.

### 3.5.3.2 Framework and Technology for SoftBoot

A) Frontend: HTML5 was used for content and structure creation, CSS3 was used for layout and styling, while JavaScript was used for interaction and dynamic functionality of the website.

B) Backend: Python had been used for server-side logic and API integration, and Flask was used for application development.

C) Speech-to-speech translation model: Speech-to-speech translation models, the base model was developed using Pytorch library because of its tensor flow computation and aiding real-time GPU processing for most deep learning based models. For transfer learning, popular pre-trained models such as Google Translatotron 2 and Facebook’s FAIRseq had been used for model development.

D) Database: The parallel speech corpora are stored using the Google data storage. For collection of user traffic on the SoftBot, Google form was used.

### 3.5.3.3 User Interface Design

A) Wireframes Creation and Website’s layout and design: The wireframes and website’s layout as well as the designs were carried out using Figma. The choice of the Figma was due to its user interface with lots of features and shapes for the design of the website and linking with Github for various operations.

B) Decision on the color scheme, typography, and imagery: The colors had been selected to give the interface a user friendly experience and ensuring easy readability of texts and other features on the page.

C Responsiveness of the website and accessibility on various devices: The SoftBot had been designed to work seamlessly on all mobile and smart devices by setting the width of the images, boxes to max-width and utilization of rem or em in place of pix for images, fonts etc. Different screen sizes had been set to cater for different smart devices.

D) Creation of a user friendly interface: The created SoftBot is user friendly with great user experience taking the design steps, color combination, and responsiveness of the page into consideration. It accepts English audios as inputs and produces two sets of outputs: Yoruba audios and texts.

### 3.5.3.4 Development of the frontend

A) Writing of the HTML, CSS, and JavaScript code to create website’s structure, styling, and functionality

The HTML 5, CSS 3, and JavaScript were utilized to develop the prototype SoftBot. The three web development tools had been chosen due to their easy accessibilities and easy usage for web site creation. The HTML tags such as <p>, <article>, <div>, <section>, <h>, <img>, <input>, etc. were used to create the contents of the page. The ensure styling, the CSS 3 was used. The inward, internal, and external styling from a new CSS file were all utilized. JavaScript was used to add some functions to the styled HTML page.

B) Use of a frontend framework like React, Angular if desired: This was not used in this work

C) Implementation of audio input and output functionality using Web Audio API:  Fill here

### 3.5.3.5 Development of the backend

Backend development for this work involved the following steps:

A) Step up the backend project:

- Install python

- Choose project directory

- Create a virtual environment

- Activate the virtual environment

- Install require packages

B) Define API endpoints

- Create a new flask App

- Define API endpoints

- Run flask App

C) Implement audio input handling

- Use a library like Flask-Uploads: This is installed using: pip install flask-uploads

- Configure Flask-Uploads: In the app.py python file, Flask-Uploads is configured to handle English audio file uploads

- Handle English audio file processing

D) Integration of the speech-to-speech translation model

- Load the developed improved transformer base and transfer learning models

- Preprocess the audio files

- Pass the preprocessed audio to the developed model

- Received the translated Yoruba audios and texts.

E) Implement audio output handling

- Use Library like PyDub: This is done using: pip install pydub

- Configure pydub: In the app.py python file, Pydub is configured to handle audio output

- Handling Yoruba audios and texts output

F) Test the backend API

- This involves using a tool like Postman.

- Sending an English audio file

- Verify the response obtained (Yoruba audios and texts)

- Test error handling

### 3.5.3.6 Writing of server-side code in Python to handle API requests and integration with speech-to-speech APIs

A) Use of backend framework like Flask: Flask is a lightweight web framework for web development. It offers easy integration of HTML, CSS and JavaScripts pages as well as images etc. It offers ability to design HTML form and its validation. Hence, this flexibility makes it a popular choice for web development application. Flask python codes were developed to render the HTML pages, integrate with the CSS and JavaScripts codes.

Implementation of API endpoints for uploading English audio files, and generating Yoruba audio files and Yoruba texts

Pytorch for integration of speech-to-speech translation model into the SoftBot: Poytorch library was used within the Python codes to integrate the speech-to-speech translation model into the SoftBot.

### 3.5.3.7 Speech-to-Speech Translation Model

The steps used to integrate the speech-to-speech translation model into the prototype SoftBot are:

A) Prepare your own model

- Ensure compatibility

- Model serialization

- Model loading: The developed base model or transfer learning model is loaded in order to utilized for translation use case on the SoftBot.

B) Integrate model with backend

- Create a model wrapper

- Implement prediction logic

- Integrate with backend API

C) Handling audio input and output

- Audio input handling

- Audio output handling

D) Test model integration

- Test prediction logic

- Test backend API

### 3.5.3.8 Conversation Management

A) Handling of user input

B) Generation of response: The SoftBot is able to generate Yoruba audios and Yoruba texts in a user friendly way.

### 3.5.3.9 Prototype Testing and Evaluation

A) Testing of website’s functionality, usability, and performance: The functionality of the developed SoftBot is put to test by sourcing users to use it for some days and user feedback was utilized to enhance the interface.

B) Debugging and fixing of any issues that arise during testing: Via the user feedback, errors were accounted for by fixing them and re-testing was carried out number of times till perfection was achieved.

C) Deploying of website to a hosting platform: The tested flask application developed was deployed using Google web application for real-time engagement by the user.

D) Configuring of SSL/TLS certificates for secure HTTPS connection: The security credentials of the deployed SoftBot was carried out to ensure it is safe from malicious attack by untended user and hackers.

### 3.5.3.10 Maintenance and Update of the Website

A) Monitoring of website performance: Real-time monitoring of the SoftBot was carried out for user traffic, user time engagement etc. and the need to enhance the interface.

B) Updating the website with new security features, bug fixes, and security patches

C) Continuous improvement of the website’s user experience, accessibility, and overall quality: This was carried out based on the collected user feedback.

# 3.6 Budget and Timeline

## 3.6.1 Research Budget

The budget for the proposed research is presented in Table 3.3:

Table 3.3: Research Budget

| S/n | ITEM DESCRIPTION | UNIT PRICE ($) | QUANTITY | TOTAL PRICE ($) |
| --- | --- | --- | --- | --- |
| 1 | Speech Data Curation |  | 11 Weeks | 10,213.10 |
| 2 | Conferences (CApIC-ACE) | 750 | 2 | 1, 500 |
| 3 | Publication (CApIC-ACE) | 750 | 2 | 4, 000 |
|  | Total |  |  | 15,713.10 |

The sum of fifteen thousand seven hundred and thirteen thousand, ten cents dollars is estimated for the realization of this proposed work.

This work is funded by the Covenant Applied Informatics and Communication Africa Center of Excellence, CApIC-ACE, and Google.

The Google funding will be utilised for curation of dataset. The analysis, pre-processing, training and prototype SoftBot development will be funded by Google and CApIC ACE.

## 3.6.2 Timelines

The Timeline for the completion of the research work and the completion due date of the research work are summarized in Table 3.5

Table 3.5: Research Timeline

# 3. 7 Conclusion

The development of speech-to-speech modelling has seen revolution and improvement for high-resourced languages like English, Spanish, French, and German. However, the African indigenous languages, including the Yoruba language, are still underdeveloped in language translation from another language to Yoruba and vice versa. This proposed work investigates how to make publicly available parallel English-Yoruba speech corpora to aid further studies in model development for other researchers. Besides, the utilisation of speech translation in medical space has not seen much progress. This is due to the sensitivity of the information or medical conversation it is to be utilised for, as the output of speech translation comes with many errors, which researchers are still working on to overcome. Hence, the utilisation of speech discretised features will be looked into as the key pre-processing algorithm for ensuring improved transformer model as well as carrying out different ablation studies to obtain the best enhanced transformer model for integration with the developed prototype SoftBot. This work will leverage the transfer learning approach to train medical speech corpus and non-medical speech corpus with the aim of obtaining a robust speech translation output which will further guarantee the utilisation of speech translation in the medical space, as well as public engagement between medical health practitioners and public thereby addressing the SDGs 3 and 10 and the IR4.

# References

Adeniyi, S. K., Ajiboye, O. J., & Ilori, J. F. (n.d.). A Morphological and Computational Analysis of Yoruba Numeral Multiplicands: Linguistic Structure and Cultural Significance.

Armaisya, D. D., Pamungkasari, P. D., Rifai, A. P., Sholihati, I. D., & Sakarkar, G. (2025). Comparison Of Feature Extraction Techniques For Long Short-Term Memory Models In Indonesian Automatic Speech Recognition. Green Intelligent Systems and Applications, 5(1), 74–92.

Ayesha, M. L., Divya, M. O., & others. (2022). Universal Speech Translator. Kristu Jayanti Journal of Computational Sciences (KJCS), 88–96.

Azterketa Eta Prozesamendua, H. (2023). Basque and Spanish Multilingual TTS Model for Speech-to-Speech Translation hap/lap.

Baevski, A., Zhou, H., Mohamed, A., & Auli, M. (2020). wav2vec 2.0: A Framework for Self-Supervised Learning of Speech Representations. https://github.com/pytorch/fairseq

Bahdanau, D., Cho, K., & Bengio, Y. (2014). Neural machine translation by jointly learning to align and translate. ArXiv Preprint ArXiv:1409.0473.

Bakenne, N. A., & Salawu, A. (2025). BBC News Yorùbá and Digital Public Sphere: Promoting Yorùbá Cultural Traditions among Youths of South-West Nigeria. E-BANGI, 22(1), 382–394.

Baral, A. (2025). Preserving Indigenous Language: Text-To-Speech System for the Myaamia Language. Miami University.

Barrault, L., Chung, Y.-A., Meglioli, M. C., Dale, D., Dong, N., Duquenne, P.-A., Elsahar, H., Gong, H., Heffernan, K., Hoffman, J., & others. (2023). SeamlessM4T-Massively Multilingual & Multimodal Machine Translation. ArXiv Preprint ArXiv:2308.11596.

Bozkurt, A. ;, Junhong, X. ;, Lambert, S. ;, Pazurek, A. ;, Crompton, H. ;, Koseoglu, S. ;, Farrow, R. ;, Bond, M. ;, Nerantzi, C. ;, Honeychurch, S. ;, Bali, M. ;, Dron, J. ;, Mir, K. ;, Stewart, B. ;, Costello, E. ;, Mason, J. ;, Stracke, C. M. ;, Romero-Hall, Bozkurt, A., … Stracke, J. (2023). Reflection from the Educational Landscape. Asian Journal of Distance Education, 18(1), 53–130. https://doi.org/10.5281/zenodo.7636568

Cattoni, R., Di Gangi, M. A., Bentivogli, L., Negri, M., & Turchi, M. (2021). MuST-C: A multilingual corpus for end-to-end speech translation. Computer Speech and Language, 66. https://doi.org/10.1016/j.csl.2020.101155

Chang, X., Watanabe, S., Delcroix, M., Ochiai, T., Zhang, W., & Qian, Y. (2025). Module-Based End-to-End Distant Speech Processing: A case study of far-field automatic speech recognition [Special Issue On Model-Based and Data-Driven Audio Signal Processing]. IEEE Signal Processing Magazine, 41(6), 39–50.

Chang, X., Yan, B., Choi, K., Jung, J., Lu, Y., Maiti, S., Sharma, R., Shi, J., Tian, J., Watanabe, S., Fujita, Y., Maekaku, T., Guo, P., Cheng, Y.-F., Denisov, P., Saijo, K., & Wang, H.-H. (2023). Exploring Speech Recognition, Translation, and Understanding with Discrete Speech Units: A Comparative Study. http://arxiv.org/abs/2309.15800

Chauhan, S., Daniel, P., Saxena, S., & Sharma, A. (2022). Fully unsupervised machine translation using context-aware word translation and denoising autoencoder. Applied Artificial Intelligence, 36(1), 2031817.

Chen, P.-J., Tran, K., Yang, Y., Du, J., Kao, J., Chung, Y.-A., Tomasello, P., Duquenne, P.-A., Schwenk, H., Gong, H., & others. (2022). Speech-to-speech translation for a real-world unwritten language. ArXiv Preprint ArXiv:2211.06474.

Cherednichenko, O. (2020). Collection and Processing of a Medical Corpus in Ukrainian.

de Martos, A. M. (2022). Deep neural networks for automatic speech-to-speech translation of open educational resources. Universitat Politècnica de València.

Desai, N., Dhameliya, K., & Desai, V. (2013). Feature extraction and classification techniques for speech recognition: A review. International Journal of Emerging Technology and Advanced Engineering, 3(12), 367–371.

Dogra, T., & Malik Student, R. (2024). The Representation of the “Queer” in Alice Oseman’s Heartstopper and Casey McQuiston One Last Stop. In Certified Journal | (Vol. 9001).

Dong, Q., Yue, F., Ko, T., Wang, M., Bai, Q., & Zhang, Y. (2022). Leveraging Pseudo-labeled Data to Improve Direct Speech-to-Speech Translation. http://arxiv.org/abs/2205.08993

Elesemoyo, I. O. (2025). An Android-Based Number Conversion System to Yorùbá Language Numeral Forms. FUOYE Journal of Pure and Applied Sciences (FJPAS), 10(1), 102–113.

Fan, Y., Qian, Y., Xie, F.-L., & Soong, F. K. (2014). TTS synthesis with bidirectional LSTM-based recurrent neural networks. Fifteenth Annual Conference of the International Speech Communication Association.

Gong, Z., Xu, X., & Zhao, Y. (2025). Tibetan–Chinese speech-to-speech translation based on discrete units. Scientific Reports, 15(1), 2592.

Grabar, N., & Cardon, R. 2019. CLEAR-Simple Corpus for Medical French. http://natalia.

Griffin, D., & Lim, J. (1984). Signal estimation from modified short-time Fourier transform. IEEE Transactions on Acoustics, Speech, and Signal Processing, 32(2), 236–243.

Gutkin, A., Demirsahin, I., Kjartansson, O., Rivera, C. E., & Túbòsún, K. (2020). Developing an open-source corpus of Yoruba speech.

Haluza, D., & Jungwirth, D. (2023). Artificial Intelligence and Ten Societal Megatrends: An Exploratory Study Using GPT-3. Systems, 11(3), 120.

Hassana, I. L., & Sanusi, M. (2019). Text to Speech Synthesis System in Yoruba Language. International Journal of Advances in Scientific Research and Engineering, 5(10), 180–191.

Hsu, W. N., Bolte, B., Tsai, Y. H. H., Lakhotia, K., Salakhutdinov, R., & Mohamed, A. (2021). Hubert: Self-supervised speech representation learning by masked prediction of hidden units. IEEE/ACM transactions on audio, speech, and language processing, 29, 3451-3460.

Hudelson, P., & Chappuis, F. (2024). Using Voice-to-Voice Machine Translation to Overcome Language Barriers in Clinical Communication: An Exploratory Study. Journal of General Internal Medicine. https://doi.org/10.1007/s11606-024-08641-w

Hujon, A. V., Singh, T. D., & Amitab, K. (2022). Transfer Learning Based Neural Machine Translation of English-Khasi on Low-Resource Settings. Procedia Computer Science, 218, 1–8. https://doi.org/10.1016/j.procs.2022.12.396

Hutchins, J. (1997). From first conception to first demonstration: the nascent years of machine translation, 1947–1954. a chronology. Machine Translation, 12, 195–252.

Javaid, M., Haleem, A., & Singh, R. P. (2023). ChatGPT for healthcare services: An emerging stage for an innovative perspective. BenchCouncil Transactions on Benchmarks, Standards and Evaluations, 3(1). https://doi.org/10.1016/j.tbench.2023.100105

Jeuris, P., & Niehues, J. (2022). Libris2s: A german-english speech-to-speech translation corpus. ArXiv Preprint ArXiv:2204.10593.

Jia, Y., Johnson, M., Macherey, W., Weiss, R. J., Cao, Y., Chiu, C.-C., Ari, N., Laurenzo, S., & Wu, Y. (2019). Leveraging weakly supervised data to improve end-to-end speech-to-text translation. ICASSP 2019-2019 IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP), 7180–7184.

Jia, Y., Ramanovich, M. T., Remez, T., & Pomerantz, R. (2022). Translatotron 2: High-quality direct speech-to-speech translation with voice preservation. International Conference on Machine Learning, 10120–10134.

Jia, Y., Weiss, R. J., Biadsy, F., Macherey, W., Johnson, M., Chen, Z., & Wu, Y. (2019). Direct speech-to-speech translation with a sequence-to-sequence model. http://arxiv.org/abs/1904.06037

Jin, Z., Finkelstein, A., Mysore, G. J., & Lu, J. (2018). FFTNet: A real-time speaker-dependent neural vocoder. 2018 IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP), 2251–2255.

Julius, O., Obagbuwa, I. C., Michael, E. B., Adebiyi, A. A., Deborah, O., & Akinrinwa, S. P. (2024). Direct English-to-Yoruba speech Translation model using Transformer with Augmented Attention Mechanism. 2024 IEEE 5th International Conference on Electro-Computing Technologies for Humanity (NIGERCON), 1–6.

Kalchbrenner, N., & Blunsom, P. (2013). Recurrent continuous translation models. Proceedings of the 2013 Conference on Empirical Methods in Natural Language Processing, 1700–1709.

Kalchbrenner, N., Elsen, E., Simonyan, K., Noury, S., Casagrande, N., Lockhart, E., Stimberg, F., Oord, A., Dieleman, S., & Kavukcuoglu, K. (2018). Efficient neural audio synthesis. International Conference on Machine Learning, 2410–2419.

Kaur, P., Bush, L. A. M., & Shi, W. (2024). Direct Punjabi to English speech translation using discrete units. http://arxiv.org/abs/2402.15967

Kharitonov, E., Copet, J., Lakhotia, K., Nguyen, T. A., Tomasello, P., Lee, A., Elkahky, A., Hsu, W.-N., Mohamed, A., Dupoux, E., & others. (2022). textless-lib: A library for textless spoken language processing. ArXiv Preprint ArXiv:2202.07359.

Kim, S.-B., Lee, S.-H., & Lee, S.-W. (2024). TranSentence: Speech-to-speech Translation via Language-agnostic Sentence-level Speech Encoding without Language-parallel Data. https://doi.org/10.1109/ICASSP48485.2024.10447331

Knowles, K. L., Patten, K. J., Durojaye, C., Dada, B. O., & McBeath, M. K. (2025). Yorùbá Speech Surrogacy with the Dùndún Talking Drum: Recognition Accuracy is a Function of Language Familiarity and Music Training. Music Perception: An Interdisciplinary Journal, 1–24.

Kocmi, Tom., Macháček, Dominik., & Bojar, O. (2022). The reality of multi-lingual machine translation.

Kong, J., Kim, J., & Bae, J. (2020). Hifi-gan: Generative adversarial networks for efficient and high fidelity speech synthesis. Advances in Neural Information Processing Systems, 33, 17022–17033.

Kumar, K., Kumar, R., De Boissiere, T., Gestin, L., Teoh, W. Z., Sotelo, J., De Brebisson, A., Bengio, Y., & Courville, A. C. (2019). Melgan: Generative adversarial networks for conditional waveform synthesis. Advances in Neural Information Processing Systems, 32.

Lee, A., Chen, P.-J., Wang, C., Gu, J., Popuri, S., Ma, X., Polyak, A., Adi, Y., He, Q., Tang, Y., Pino, J., & Hsu, W.-N. (2021). Direct speech-to-speech translation with discrete units. http://arxiv.org/abs/2107.05604

Li, J., Lavrukhin, V., Ginsburg, B., Leary, R., Kuchaiev, O., Cohen, J. M., Nguyen, H., & Gadde, R. T. (2019a). Jasper: An end-to-end convolutional neural acoustic model. ArXiv Preprint ArXiv:1904.03288.

Li, J., Lavrukhin, V., Ginsburg, B., Leary, R., Kuchaiev, O., Cohen, J. M., Nguyen, H., & Gadde, R. T. (2019b). Jasper: An end-to-end convolutional neural acoustic model. ArXiv Preprint ArXiv:1904.03288.

Li, R., Wang, X., & Yu, H. (2020). MetaMT, a meta learning method leveraging multiple domain data for low resource machine translation. Proceedings of the AAAI Conference on Artificial Intelligence, 34(05), 8245–8252.

Li, X., Jia, Y., & Chiu, C.-C. (2022). Textless Direct Speech-to-Speech Translation with Discrete Speech Representation. http://arxiv.org/abs/2211.00115

Limbu, S. H. (2020). Direct Speech to Speech Translation Using Machine Learning. http://www.teknat.uu.se/student

Liu, R., Sisman, B., Li, J., Bao, F., Gao, G., & Li, H. (2020). Teacher-student training for robust tacotron-based tts. ICASSP 2020-2020 IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP), 6274–6278.

Luong, M.-T., Pham, H., & Manning, C. D. (2015). Effective approaches to attention-based neural machine translation. ArXiv Preprint ArXiv:1508.04025.

McArthur, B., & Richardson, S. D. (2023). Direct Speech to Speech Translation: Model Improvements.

Medeiros, E. F. (2023). Deep learning for speech to text transcription for the portuguese language. Universidade de Évora.

Meyer, J., Adelani, D. I., Casanova, E., Öktem, A., Weber, D. W. J., Kabongo, S., Salesky, E., Orife, I., Leong, C., Ogayo, P., Emezue, C., Mukiibi, J., Osei, S., Agbolo, A., Akinode, V., Opoku, B., Olanrewaju, S., Alabi, J., & Muhammad, S. (2022). BibleTTS: a large, high-fidelity, multilingual, and uniquely African speech corpus. http://arxiv.org/abs/2207.03546

Murphy, K. P. (2022). Probabilistic machine learning: an introduction. MIT Press.

Nachmani, E., Levkovitch, A., Ding, Y., Asawaroengchai, C., Zen, H., & Ramanovich, M. T. (2023). Translatotron 3: Speech to Speech Translation with Monolingual Data. http://arxiv.org/abs/2305.17547

Nallabala, N. K., Souprayen, B., Ramasamy, M., Penumarti, S. K., & Navuluri, N. C. (2025). An Efficient Speech Synthesizer: A Hybrid Monotonic Architecture for Text-to-speech via VAE & LPC-Net with Independent Sentence Length. Circuits, Systems, and Signal Processing, 1–25.

Niranjan, A., Sharma, M., Gutha, S. B. C., & Shaik, M. A. B. (2020). End-to-End Whisper to Natural Speech Conversion using Modified Transformer Network. http://arxiv.org/abs/2004.09347

Nyetanyane, J., & Masinde, M. (2019). UmobiTalk: Ubiquitous Mobile Speech Based Translator for Sesotho Language. Emerging Technologies for Developing Countries: Second EAI International Conference, AFRICATEK 2018, Cotonou, Benin, May 29–30, 2018, Proceedings 2, 93–106.

Olagunju, O. B., Abiola, O. B., Saka-Balogun, O. Y., Badeji-Ajisafe, B., Oguntimilehin, A., Adewole, L. O., Adigun, K. A., Obamiyi, S. E., & Oluwatoki, T. G. (2024). A Transformer-Based Model for English to Yoruba Machine Translation. 2024 IEEE 5th International Conference on Electro-Computing Technologies for Humanity (NIGERCON), 1–5.

Olatunji, T., Afonja, T., Yadavalli, A., Emezue, C. C., Singh, S., Dossou, B. F. P., Osuchukwu, J., Osei, S., Tonja, A. L., Etori, N., & Mbataku, C. (2023). AfriSpeech-200: Pan-African Accented Speech Dataset for Clinical and General Domain ASR. http://arxiv.org/abs/2310.00274

Oord, A., Li, Y., Babuschkin, I., Simonyan, K., Vinyals, O., Kavukcuoglu, K., Driessche, G., Lockhart, E., Cobo, L., Stimberg, F., & others. (2018). Parallel wavenet: Fast high-fidelity speech synthesis. International Conference on Machine Learning, 3918–3926.

Oord, A. van den, Dieleman, S., Zen, H., Simonyan, K., Vinyals, O., Graves, A., Kalchbrenner, N., Senior, A., & Kavukcuoglu, K. (2016). Wavenet: A generative model for raw audio. ArXiv Preprint ArXiv:1609.03499.

O’Shaughnessy, D. (2008). Automatic speech recognition: History, methods, and challenges. Pattern Recognition, 41(10), 2965–2979.

Overmann, K. A. (2025). Yoruba Numbers. In Cultural Number Systems: A Sourcebook (pp. 59–64). Springer.

Oyewole, J. A., Oyewole, C. F., Ayomide, P. A. O., & Oyewole, J. A. (2025). Language Practices and Social Integration among Multi-Ethnic Youth: A Yoruba Perspective. Yoruba Studies Review, 10(1), 137–164.

Panayiotou, A., Hwang, K., Williams, S., Chong, T. W. H., LoGiudice, D., Haralambous, B., Lin, X., Zucchi, E., Mascitti-Meuter, M., Goh, A. M. Y., You, E., & Batchelor, F. (2020). The perceptions of translation apps for everyday health care in healthcare workers and older people: A multi-method study. Journal of Clinical Nursing, 29(17–18), 3516–3526. https://doi.org/10.1111/jocn.15390

Ping, W., Peng, K., Gibiansky, A., Arik, S. O., Kannan, A., Narang, S., Raiman, J., & Miller, J. (2017). Deep voice 3: Scaling text-to-speech with convolutional sequence learning. ArXiv Preprint ArXiv:1710.07654.

Prenger, R., Valle, R., & Catanzaro, B. (2019). Waveglow: A flow-based generative network for speech synthesis. ICASSP 2019-2019 IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP), 3617–3621.

Qian, K., Huang, R., Bao, Z., Tan, Y., Zhao, Z., Sun, M., Hu, B., Schuller, B. W., & Yamamoto, Y. (2023). Detecting somatisation disorder via speech: introducing the Shenzhen Somatisation Speech Corpus. Intelligent Medicine. https://doi.org/10.1016/j.imed.2023.03.001

Rabiner, L. R., & Schafer, R. W. (2007). the essence of knowledge Introduction to Digital Speech Processing Introduction to Digital Speech Processing.

Rabiner, L. R., Schafer, R. W., & others. (2007). Introduction to digital speech processing. Foundations and Trends®in Signal Processing, 1(1--2), 1–194.

Radford, A., Kim, J. W., Xu, T., Brockman, G., McLeavey, C., & Sutskever, I. (2023). Robust speech recognition via large-scale weak supervision. International Conference on Machine Learning, 28492–28518.

Ranathunga, S., Lee, E.-S. A., Skenduli, M. P., Shekhar, R., Alam, M., & Kaur, R. (2021). Neural Machine Translation for Low-Resource Languages: A Survey. http://arxiv.org/abs/2106.15115

Rapp, R. (2021). Similar language translation for Catalan, Portuguese, and Spanish using Marian NMT. Proceedings of the Sixth Conference on Machine Translation, 292–298.

Rubenstein, P. K., Asawaroengchai, C., Nguyen, D. D., Bapna, A., Borsos, Z., Quitry, F. de C., Chen, P., Badawy, D. El, Han, W., Kharitonov, E., Muckenhirn, H., Padfield, D., Qin, J., Rozenberg, D., Sainath, T., Schalkwyk, J., Sharifi, M., Ramanovich, M. T., Tagliasacchi, M., … Frank, C. (2023). AudioPaLM: A Large Language Model That Can Speak and Listen. http://arxiv.org/abs/2306.12925

Rubenstein, P. K., Asawaroengchai, C., Nguyen, D. D., Bapna, A., Borsos, Z., Quitry, F. de C., Chen, P., Badawy, D. El, Han, W., Kharitonov, E., & others. (2023). Audiopalm: A large language model that can speak and listen. ArXiv Preprint ArXiv:2306.12925.

Sandeep Gupta, S., & Ramakant Shirodkar, V. (2022). A study on the techniques for speech to speech translation. www.irjet.net

Shen, J., Pang, R., Weiss, R. J., Schuster, M., Jaitly, N., Yang, Z., Chen, Z., Zhang, Y., Wang, Y., Skerrv-Ryan, R., & others. (2018). Natural tts synthesis by conditioning wavenet on mel spectrogram predictions. 2018 IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP), 4779–4783.

Shi, J., Tang, Y., Lee, A., Inaguma, H., Wang, C., Pino, J., & Watanabe, S. (2023). Enhancing Speech-to-Speech Translation with Multiple TTS Targets. http://arxiv.org/abs/2304.04618

Silaban, W. Y., Lestari, M., Desi, W. ;, Barumbu, N., Kawet, K., Yosua, ;, Sunkudon, A., Darmawan, ;, & Winoto, E. (2023). Comparative Study of Post-Marriage Nationality of Women in Legal Systems of Different Countries International Journal of Multicultural and Multireligious Understanding History of English Become an International Language. https://doi.org/10.18415/ijmmu.v10i2.4359

Suikkila, J. (2025). Museum wayfinding with AI-spoken voice guidance.

Sutskever, I., Vinyals, O., & Le, Q. V. (2014). Sequence to sequence learning with neural networks. Advances in Neural Information Processing Systems, 27.

Tu, T., Palepu, A., Schaekermann, M., Saab, K., Freyberg, J., Tanno, R., Wang, A., Li, B., Amin, M., Tomasev, N., Azizi, S., Singhal, K., Cheng, Y., Hou, L., Webson, A., Kulkarni, K., Mahdavi, S. S., Semturs, C., Gottweis, J., … Natarajan, V. (2024). Towards Conversational Diagnostic AI. http://arxiv.org/abs/2401.05654

Valin, J.-M., & Skoglund, J. (2019). LPCNet: Improving neural speech synthesis through linear prediction. ICASSP 2019-2019 IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP), 5891–5895.

Vaswani, A., Shazeer, N., Parmar, N., Uszkoreit, J., Jones, L., Gomez, A. N., Kaiser, Ł., & Polosukhin, I. (2017). Attention is all you need. Advances in Neural Information Processing Systems, 30.

Ververidis, D., & Kotropoulos, C. (2006). Emotional speech recognition: Resources, features, and methods. Speech Communication, 48(9), 1162–1181.

Wadhawan, A. (2023). SIMULTANEOUS SPEECH TO SPEECH TRANSLATION.

Wang, Y., Skerry-Ryan, R. J., Stanton, D., Wu, Y., Weiss, R. J., Jaitly, N., Yang, Z., Xiao, Y., Chen, Z., Bengio, S., & others. (2017). Tacotron: Towards end-to-end speech synthesis. ArXiv Preprint ArXiv:1703.10135.

White, A., & Watson, C. (2025). Towards the Detection of Speech Deepfakes for Scam Prevention. ResearchSpace@ Auckland.

Workshop, B., :, Scao, T. Le, Fan, A., Akiki, C., Pavlick, E., Ilić, S., Hesslow, D., Castagné, R., Luccioni, A. S., Yvon, F., Gallé, M., Tow, J., Rush, A. M., Biderman, S., Webson, A., Ammanamanchi, P. S., Wang, T., Sagot, B., … Wolf, T. (2022). BLOOM: A 176B-Parameter Open-Access Multilingual Language Model. http://arxiv.org/abs/2211.05100

Wu, Y., Schuster, M., Chen, Z., Le, Q. V, Norouzi, M., Macherey, W., Krikun, M., Cao, Y., Gao, Q., Macherey, K., & others. (2016). Google’s neural machine translation system: Bridging the gap between human and machine translation. ArXiv Preprint ArXiv:1609.08144.

Xue, L., Constant, N., Roberts, A., Kale, M., Al-Rfou, R., Siddhant, A., Barua, A., & Raffel, C. (2020). mT5: A massively multilingual pre-trained text-to-text transformer. ArXiv Preprint ArXiv:2010.11934.

Yamamoto, R., Song, E., & Kim, J.-M. (2020). Parallel WaveGAN: A fast waveform generation model based on generative adversarial networks with multi-resolution spectrogram. ICASSP 2020-2020 IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP), 6199–6203.

Yao, S., & Poonpon, K. (2025). The Effects of Text-to-Speech Dictation on Chinese Elementary Students’ Listening and Spelling Skills at a Primary School in China. Theory and Practice in Language Studies, 15(1), 294–306.

Zhang, A., Lipton, Z. C., Li, M., & Smola, A. J. (2021). Dive into deep learning. ArXiv Preprint ArXiv:2106.11342.